# MPMC — Module 2 · PART 1: 8086 ARCHITECTURE — EXAM-READY NOTES

### Topic: Generations · Functional blocks · Pin diagram & signals · BIU–EU architecture · Pipelining · Registers · Flags · Memory segmentation & physical address · Interrupts

Source: Module 2 lecture slides (ECE3004 – Microprocessors and Microcontrollers).

Module 2 is split into 3 files — use them together, nothing is left out:

•  Part 1 (this file) → Architecture: generations, pins, BIU/EU, registers, flags, physical address, interrupts

•  Part 2 → MPMC_Module2_Part2_Addressing_Modes.md — all 12 addressing modes

•  Part 3 → MPMC_Module2_Part3_Instruction_Set.md — instruction set, assembler directives, procedures/macros, programs

•  (All-in-one master copy: MPMC_Module2_Exam_Notes.md)

## CONTENTS OF THIS FILE

1.  Generations of microprocessors

2.  General microprocessor functional blocks

3.  8086 microprocessor — overview & salient features

4.  8086 pin diagram and signals (common / minimum / maximum mode)

5.  Inside the 8086 — BIU + EU architecture and pipelining

6.  Registers of 8086 and the flag register

7.  Memory segmentation & physical address generation

8.  8086 interrupts — sources, processing, IVT, groups, types 0–4

9.  Most likely exam questions (architecture)

10.  Last-minute revision cram sheet (architecture)

# 1. GENERATIONS OF MICROPROCESSORS

| Generation | Period | Technology | Word size / Pins | Key features | Example |
| --- | --- | --- | --- | --- | --- |
| First | 1971 – 1973 | PMOS (not TTL compatible) | 4-bit → 16 pins; 8- & 16-bit → 40 pins | Signals had to be multiplexed due to pin limitations | Intel 4004/4040 |
| Second | 1973 onward | NMOS (faster, higher density, TTL compatible) | 4 / 8 / 16-bit → 40 pins | Ability to address large memory spaces and I/O ports, greater levels of subroutine nesting, better interrupt handling | Intel 8085 (8-bit) |
| Third | 1978 onward | HMOS (faster, higher packing density) | 16-bit → 40 / 48 / 64 pins | Easier to program, dynamically relocatable programs, multiply/divide arithmetic hardware, more powerful interrupt handling, flexible I/O port addressing | Intel 8086 (16-bit) |
| Fourth | 1980s | HCMOS (low-power version of HMOS) | 32-bit | Physical memory space 2²⁴ = 16 MB, virtual memory space 2⁴⁰ = 1 TB, floating-point hardware, increased number of addressing modes | Intel 80386 |
| Fifth | — | — | 32/64-bit | Advanced pipelining, superscalar | Pentium |

Memory trick: P-N-H-H-P = PMOS → NMOS → HMOS → HCMOS → Pentium.

# 2. GENERAL MICROPROCESSOR FUNCTIONAL BLOCKS

| Block | Function |
| --- | --- |
| ALU (Computational Unit) | Performs arithmetic and logic operations |
| Flag Register | Stores various conditions of the results as status bits called flags |
| Register array / internal memory | Internal storage of data |
| PC / IP | Generates the address of the instructions to be fetched from memory and sends it through the address bus |
| Instruction decoding unit | Decodes instructions; sends information to the timing and control unit |
| Timing and control unit | Generates control signals for internal and external operations of the microprocessor |

Buses: Control bus, Address bus, Data bus interconnect these blocks with memory and I/O.

# 3. 8086 MICROPROCESSOR — OVERVIEW & SALIENT FEATURES

•  First 16-bit processor released by INTEL in the year 1978.

•  Originally HMOS, now manufactured using the HMOS III technique.

•  Approximately 29,000 transistors, 40-pin DIP, +5 V supply.

•  Does not have an internal clock → needs an external asymmetric clock source with 33 % duty cycle (supplied by the 8284A clock generator).

•  20-bit address to access memory → can address up to 2²⁰ = 1 megabyte of memory space.

•  16-bit data bus; word size = 16 bits.

•  Two operating modes: Minimum mode (no coprocessors, single processor) and Maximum mode (multiprocessor/coprocessor configurations).

Quick comparison table (frequently asked):

| Feature | 8085 | 8086 |
| --- | --- | --- |
| Word length | 8-bit | 16-bit |
| Address bus | 16-bit | 20-bit |
| Memory | 64 KB | 1 MB |
| Clock | 3 MHz | 5 / 8 / 10 MHz (version dependent) |
| Multiplexing | Lower address/data | Address/data + status multiplexed |
| Pipelining | No | Yes (6-byte instruction queue) |

# 4. 8086 PIN DIAGRAM AND SIGNALS (40-pin DIP)

Pin layout (learn the groups):

| Pins | Name |
| --- | --- |
| 1, 20 | GND (ground) |
| 2–16, 39 | AD0–AD15 (address/data bus, bidirectional, multiplexed) |
| 35–38 | A16/S3, A17/S4, A18/S5, A19/S6 |
| 17 | NMI |
| 18 | INTR |
| 19 | CLK |
| 21 | RESET |
| 22 | READY |
| 23 | TEST |
| 24 | QS1 |
| 25 | QS0 (max mode) / ALE (min mode) |
| 26 | S0 (max) / DEN (min) |
| 27 | S1 (max) / DT/R (min) |
| 28 | S2 (max) / M/IO (min) |
| 29 | LOCK (max) / WR (min) |
| 30 | RQ/GT1 (max) / HLDA (min) |
| 31 | RQ/GT0 (max) / HOLD (min) |
| 32 | RD |
| 33 | MN/MX (minimum/maximum mode select) |
| 34 | BHE/S7 |
| 40 | VCC |

Exam tip: The same pin carries different names in minimum and maximum modes — this is the most repeated pin question.

## 4.1 Common Signals (both modes)

| Signal | Direction | Description |
| --- | --- | --- |
| AD0–AD15 | Bidirectional, multiplexed | Low-order address bus multiplexed with data. Written A0–A15 when carrying memory addresses, D0–D15 when carrying data. |
| A16/S3 – A19/S6 | Output, multiplexed | High-order address bus multiplexed with status signals. |
| BHE/S7 (active low) | Output | Bus High Enable — enables data onto the most significant half of the data bus (D8–D15). 8-bit devices connected to the upper half of the data bus use BHE. Multiplexed with status signal S7. |
| MN/MX | Input | Indicates the mode the processor is to operate in: High (1) → Minimum mode; Low (0) → Maximum mode. |
| RD (active low) | Output | Read operation; active low. |
| READY | Input, active HIGH | Acknowledgement from a slow device or memory that it has completed the data transfer. The signal made available by the devices is synchronised by the 8284A clock generator to provide the ready input to the 8086. |
| RESET | Input, active HIGH | Causes the processor to immediately terminate its present activity. Must be active HIGH for at least four clock cycles. |
| CLK | Input | Provides the basic timing for processor operation and bus control activity. An asymmetric square wave with 33 % duty cycle. |
| INTR | Input, active HIGH, triggered | Interrupt Request — sampled during the last clock cycles of each instruction to determine the availability of the request. If any interrupt request is pending, the processor enters the interrupt acknowledge cycle. Internally synchronised. |
| NMI | Input | Non-maskable interrupt (positive-edge signal). |
| TEST | Input | Used with the WAIT instruction; the processor waits while TEST is inactive. |

Note: The 8086 does not have an on-chip clock generation circuit → the 8284 clock generator chip is connected to the CLK pin of the 8086.

## 4.2 Minimum-Mode Signals (MN/MX = 1)

| Signal | Description |
| --- | --- |
| ALE (Address Latch Enable) | Used to demultiplex the address and data lines using external latches (8282). A positive pulse is generated each time the processor begins any operation and indicates the availability of a valid address on the address/data lines. Also used to differentiate memory access and I/O access. |
| DEN (Data Enable) | Output signal from the processor used as output enable for the transceivers (8286). The transceiver is a device used to separate data from the address/data bus. |
| DT/R (Data Transmit/Receive) | Output signal from the processor to control the direction of data flow through the data transceivers. |
| M/IO | Used to differentiate memory access and I/O access: for memory reference instructions it is high; for IN and OUT instructions it is low. |
| WR (active low) | Write control signal; asserted low whenever the processor writes data to memory or I/O port. |
| INTA (active low) | Interrupt Acknowledge — when the interrupt request is accepted by the processor, the output is low on this line. |
| HOLD | Input signal to the processor from the bus masters as a request to grant control of the bus; usually used by the DMA controller to get control of the bus. |
| HLDA (Hold Acknowledge) | Acknowledge signal by the processor to the bus master requesting control of the bus through HOLD. Asserted high when the processor accepts HOLD. |

Demultiplexing hardware (very common exam figure): the 8282 latch (strobed by ALE) separates A0–A19 (and produces BHE), while the 8286 transceiver (controlled by DT/R and DEN) handles D0–D15. This converts the multiplexed AD bus into separate address and data buses.

## 4.3 Maximum-Mode Signals (MN/MX = 0)

| Signal | Description |
| --- | --- |
| S0, S1, S2 (active low) | Status signals used by the 8288 bus controller to generate bus timing and control signals. |
| QS0, QS1 | Queue Status — the processor provides the status of the queue on these lines; external devices can track the internal status of the queue. |
| LOCK (active low) | Locks the bus during the next instruction so other bus masters cannot take the bus. |
| RQ/GT0, RQ/GT1 (Request/Grant) | Bidirectional pins used to request and grant bus access by other processors (e.g. the 8087 coprocessor). |

S2 S1 S0 → Machine Cycle (MUST MEMORISE):

| S2 | S1 | S0 | Machine Cycle |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Interrupt acknowledge |
| 0 | 0 | 1 | Read I/O port |
| 0 | 1 | 0 | Write I/O port |
| 0 | 1 | 1 | Halt |
| 1 | 0 | 0 | Code access (instruction fetch) |
| 1 | 0 | 1 | Read memory |
| 1 | 1 | 0 | Write memory |
| 1 | 1 | 1 | Passive / Inactive |

QS1 QS0 → Queue operation:

| QS1 | QS0 | Queue operation |
| --- | --- | --- |
| 0 | 0 | No operation |
| 0 | 1 | First byte of an opcode from queue |
| 1 | 0 | Empty the queue |
| 1 | 1 | Subsequent byte from queue |

## 4.4 Minimum vs Maximum Mode

| Point | Minimum Mode | Maximum Mode |
| --- | --- | --- |
| MN/MX pin | High (1) | Low (0) |
| Coprocessor use | Does not associate with any coprocessor; cannot be used for multiprocessor systems | Works in multiprocessor / coprocessor configuration |
| Bus controller | Not needed — 8086 gives all signals itself | 8288 bus controller required |
| Signals generated | ALE, DEN, DT/R, M/IO, WR, INTA, HOLD, HLDA | S0–S2, QS0–QS1, LOCK, RQ/GT0–RQ/GT1 |
| Typical use | Single-processor small systems | Multiprocessor systems (8086 + 8087 + 8089 etc.) |

# 5. INSIDE THE 8086 — BIU + EU ARCHITECTURE AND PIPELINING

## 5.1 Pipelining

•  Two ways to make a CPU process information faster:

1.  Increase the working frequency — technology dependent.

2.  Change the internal architecture of the CPU ← what Intel did.

•  Pipelining = allowing the CPU to fetch and execute at the same time.

•  Intel implemented pipelining by splitting the internal structure of the 8088/8086 into two sections that work simultaneously:

•  Execution Unit (EU) — executes instructions previously fetched.

•  Bus Interface Unit (BIU) — accesses memory and peripherals.

•  BIU and EU functions are separate → instruction fetch overlaps execution → speed-up.

## 5.2 Bus Interface Unit (BIU)

Functions: fetches instructions, reads data from memory and I/O ports, writes data to memory and I/O ports.

Contains:

1.  Dedicated adder to generate the 20-bit address.

2.  Four 16-bit segment registers — CS, DS, SS, ES.

3.  Instruction Pointer (IP).

4.  6-byte instruction queue (FIFO).

Segment registers:

| Register | Points to | Offset used | Notes |
| --- | --- | --- | --- |
| CS (Code Segment) | Base/start of the current code segment | IP | 16-bit. BIU computes the 20-bit physical address by shifting CS 4 bits left and adding IP. All instructions of a program are relative to CS × 16 plus the offset provided by IP. |
| DS (Data Segment) | Current data segment | SI / DI / 16-bit displacement | 16-bit. Operands for most instructions are fetched from this segment. |
| SS (Stack Segment) | Current stack | SP (PUSH/POP), BP (based addressing) | 16-bit. Physical stack address = SS × 16₁₀ + SP; in based addressing mode the address comes from SS and BP. |
| ES (Extra Segment) | Extra segment — holds data in excess of 64 K pointed to by DS | DI (string destination) | 16-bit. String instructions use ES:DI to determine the destination address. |

Memory segmentation facts:

•  The 8086's 1 MB memory is divided into segments of up to 64 KB each.

•  Programs obtain access to code and data in the segments by changing the segment register content to point to the desired segments.

•  The 8086 can directly address four segments (256 KB within the 1 MB) at any particular time.

Instruction Pointer (IP):

•  16-bit; always points to the next instruction to be executed within the currently executing code segment.

•  Contains the 16-bit offset address pointing to the next instruction code within the 64 KB code segment area.

•  Its content is automatically incremented as the execution of the next instruction takes place.

Instruction Queue:

•  A group of First-In-First-Out (FIFO) registers in which up to 6 bytes of instruction code are pre-fetched from memory ahead of time.

•  Done to speed up execution by overlapping instruction fetch with execution.

•  This mechanism is known as pipelining.

## 5.3 Execution Unit (EU)

Functions: decodes and executes instructions. A decoder in the EU control system translates instructions.

Contains:

•  16-bit ALU for performing arithmetic and logic operations.

•  Four general-purpose registers (AX, BX, CX, DX) — each 16-bit.

•  Pointer registers — SP, BP (16-bit).

•  Index registers — SI, DI (16-bit).

•  Flag register.

8-bit splitting: some 16-bit registers can be used as two 8-bit registers:

| 16-bit | → 8-bit halves |
| --- | --- |
| AX | AH, AL |
| BX | BH, BL |
| CX | CH, CL |
| DX | DH, DL |

Exam one-liner: BIU fetches, EU executes — and both work simultaneously → pipelining.

# 6. REGISTERS OF 8086 AND THE FLAG REGISTER

## 6.1 Register groups

| Sl. | Type | Width | Name of register |
| --- | --- | --- | --- |
| 1 | General purpose | 16-bit | AX, BX, CX, DX |
|  |  | 8-bit | AL, AH, BL, BH, CL, CH, DL, DH |
| 2 | Pointer | 16-bit | SP, BP |
| 3 | Index | 16-bit | SI, DI |
| 4 | Instruction Pointer | 16-bit | IP |
| 5 | Segment | 16-bit | CS, DS, SS, ES |
| 6 | Flag (PSW) | 16-bit | Flag register |

## 6.2 Registers and their special functions

| Register | Name of the register | Special function |
| --- | --- | --- |
| AX | 16-bit Accumulator | Stores the 16-bit results of arithmetic and logic operations |
| AL | 8-bit Accumulator | Stores the 8-bit results of arithmetic and logic operations |
| BX | Base register | Used to hold the base value in base addressing mode to access memory data. Only general-purpose register whose contents can be used for addressing the 8086 memory; all memory references using BX default to DS. |
| CX | Count register | Used to hold the count value in SHIFT, ROTATE and LOOP instructions |
| DX | Data register | Used to hold data for multiplication and division operations; also holds the 16-bit port address in indirect I/O |
| SP | Stack Pointer | Holds the offset address of the top of stack memory |
| BP | Base Pointer | Holds the base value in base addressing using the SS register to access data from stack memory |
| SI | Source Index | Holds the index value of the source operand (data) for string instructions |
| DI | Destination Index | Holds the index value of the destination operand (data) for string operations |

Extra points from the slides:

•  AX/AL — the I/O instructions use AX or AL for inputting/outputting 16- or 8-bit data to/from an I/O port; multiplication and division instructions also use AX or AL.

•  BX is the only general-purpose register usable for addressing memory (default segment DS).

•  CX example: the instruction LOOP START automatically decrements CX by 1 without affecting flags and checks if CX = 0. If it is zero, the 8086 executes the next instruction; otherwise the 8086 branches to the label START.

•  SP and BP are used to access data in the stack segment. SP is used as an offset from the current SS during execution of instructions involving the stack; its contents are automatically updated (incremented/decremented) by PUSH or POP. BP contains an offset address in the current SS, used by instructions utilising the based addressing mode.

•  SI and DI are used in indexed addressing. Instructions that process data strings use SI and DI together with DS and ES respectively to distinguish between the source and destination addresses.

## 6.3 Flag Register (PSW) — bit by bit

| Bit | Flag | Name | Set (=1) when… |
| --- | --- | --- | --- |
| 0 | CF | Carry Flag | There is a carry out of MSB in case of addition, or a borrow in case of subtraction |
| 2 | PF | Parity Flag | The lower byte of the result contains an even number of 1's (odd number of 1's → set to zero) |
| 4 | AF | Auxiliary Carry Flag | There is a carry from the lowest nibble (bit 3) during addition, or a borrow from bit 3 during subtraction |
| 6 | ZF | Zero Flag | The result of the computation or comparison performed by an instruction is zero |
| 7 | SF | Sign Flag | The result of any computation is negative |
| 8 | TF | Trap Flag | The processor enters the single-step execution mode by generating internal interrupts after the execution of each instruction |
| 9 | IF | Interrupt Flag | Causes the 8086 to recognise external maskable interrupts; clearing IF disables these interrupts |
| 10 | DF | Direction Flag | Used by string manipulation instructions. If DF = 0, the string is processed from the lowest address to the highest (auto-incrementing); if DF = 1, from the highest address towards the lowest (auto-decrementing) |
| 11 | OF | Overflow Flag | An overflow occurs — i.e. the result of a signed operation is too large to be accommodated in the destination register (more than 7 bits for an 8-bit signed operation, more than 15 bits for a 16-bit signed operation) |

Bit layout (memorise this):

15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0 OF DF IF TF SF ZF AF PF CF

Bits 1, 3, 5, 12, 13, 14, 15 are undefined/unused in the 8086.

# 7. MEMORY SEGMENTATION & PHYSICAL ADDRESS GENERATION

Why needed? The 8086 has 20 address lines (→ 2²⁰ = 1 M byte of memory) but the largest register is only 16 bits.

Definitions:

•  Physical Address: the actual address of a byte in memory, i.e. the value which goes out onto the address bus (20 bits).

•  Memory addresses are represented in the form Segment : Offset — e.g. 89AB:F012.

•  16 bytes of contiguous memory = one "paragraph" (hence segment × 16₁₀).

Formula:

Physical Address = (Segment register × 16₁₀) + Offset

i.e. shift the segment register content one hexadecimal place to the left (same as appending 0) and add the required offset to form the 20-bit address.

Worked example — 89AB:F012 → 98AC2H

Segment 89AB → 89AB0 (paragraph to byte: 89AB × 10H = 89AB0) Offset F012 → F012 (offset is already in byte units) ----- Absolute (physical) address 98AC2 H

Typical segment : offset pairs:

| Access type | Segment | Offset |
| --- | --- | --- |
| Instruction fetch (code) | CS | IP |
| Stack instructions (PUSH/POP) | SS | SP |
| Based addressing (stack data) | SS | BP |
| Data (default) | DS | BX / SI / DI / displacement |
| String source | DS | SI |
| String destination | ES | DI |

Segment register memory map (slide diagram): four segment registers hold the upper 16 bits of the starting addresses of the four memory segments the 8086 is working with at any particular time. Example from the slides: with DS = 2000H, CS = 3000H, SS = 5000H and ES = 7000H, memory shows the Data Segment at 20000H–2FFFFH, Code Segment at 30000H–3FFFFH, Stack Segment at 50000H–5FFFFH and Extra Segment at 70000H–7FFFFH — each 64 KB long, inside the 1 MB space (up to FFFFFH).

Notation used in the slides (reproduce it in the exam):

•  EA = Effective Address (the offset part)

•  BA = Base Address = (segment register) × 16₁₀

•  MA = Memory Address = BA + EA

The detailed use of these three quantities — with full worked examples — is in Part 2 (Addressing Modes).

# 8. 8086 INTERRUPTS

## 8.1 Sources of Interrupts (3 types)

1.  An external signal applied to the NMI or INTR input pin → (Hardware interrupt)

2.  Execution of an INTn (n = 00H–FFH) instruction → (Software interrupt)

3.  Interrupt caused by some error condition produced in the 8086 instruction execution process — divide by zero, overflow errors, etc.

Interrupt classification chart (from slides):

8086 Interrupts ├── Hardware Interrupts │ ├── Maskable Interrupt (INTR) │ │ → The programmer CAN choose to mask specific interrupts │ │ and re-enable them later │ └── Non-maskable Interrupt (NMI) │ → The programmer CANNOT control when a non-maskable │ interrupt is served │ → The processor has to stop the main program to execute │ the NMI Service Routine └── Software Interrupts (INT n) → 256 types of software interrupts, INT 00 to INT FF

•  INTR (pin 18) — where maskable hardware interrupts are applied.

•  NMI (pin 17) — non-maskable interrupt.

Uses of Non-Maskable Interrupts:

•  Used during power failure

•  Used during critical response times

•  Used during non-recoverable hardware errors

•  Used as a watchdog interrupt

•  Used during memory parity errors

## 8.2 Interrupt Processing — Steps performed by the 8086

If an interrupt has been requested, the 8086 processes it by performing the following series of steps:

1.  Pushes the content of the flag register onto the stack to preserve the status of IF and TF flags, by decrementing the stack pointer (SP) by 2.

2.  Disables the INTR interrupt by clearing IF in the flag register.

3.  Resets TF in the flag register, to disable the single-step (trap) interrupt.

4.  Pushes the content of the Code Segment (CS) register onto the stack by decrementing SP by 2.

5.  Pushes the content of the Instruction Pointer (IP) onto the stack by decrementing SP by 2.

6.  Performs an indirect far jump to the start of the Interrupt Service Routine (ISR) corresponding to the received interrupt.

Flow of steps (slide version):

Main Program → Executes the Interrupt instruction → Jumps to the Interrupt Vector Table → Takes the CS and IP in the Vector Table → Pushes the existing CS and IP on the Stack → Loads the new CS and IP → Jumps to the ISR → Executes ISR → Comes back and continues the Main Program

Hardware action sequence:

Interrupt → Push flag register → Clear IF and TF → Push CS and IP → Load CS and IP → [Interrupt Service Routine (ISR) … ends with IRET] → Pop IP and CS → Pop flags register → main program continues

Remember the order: push FLAGS → CS → IP; IRET pops IP → CS → FLAGS.

## 8.3 Interrupt Vector Table (IVT)

•  Located at the bottom of memory: addresses 00000H – 003FFH (1 KB).

•  Holds 256 interrupt vectors, each vector = 4 bytes (2 bytes for IP + 2 bytes for CS).

•  For an interrupt of type n, the ISR address is stored at:

Offset = Type number × 4

Example: INT 02H → Offset = 02 × 4 = 08 = 00008H

Layout of one vector (Type 0 shown in the slides):

| Memory address | Content |
| --- | --- |
| 00000H | IP low byte |
| 00001H | IP high byte |
| 00002H | CS low byte |
| 00003H | CS high byte |

IVT layout (memory addresses in hex):

| Address | Vector |
| --- | --- |
| 00000H–00003H | Type 00H — Divide by Zero |
| 00004H–00007H | Type 01H — Trap / Single step |
| 00008H–0000BH | Type 02H — NMI |
| 0000CH–0000FH | Type 03H — Break point |
| 00010H–00013H | Type 04H — Overflow |
| 00014H–0007FH | Type 05H … — Reserved |
| 00080H–00083H | Type 20H — Available |
| 00084H–00087H | Type 21H — Available |
| … | … |
| 003FCH–003FFH | Type FFH — Available |

Order inside each 4-byte vector: lower 2 bytes = IP, upper 2 bytes = CS.

## 8.4 The 256 Interrupts are Divided into 3 Groups

| Group | Types | Purpose |
| --- | --- | --- |
| 1 | Type 00 to Type 04 | Used for fixed operations → called dedicated interrupts (5 interrupts) |
| 2 | Type 05 to Type 31 | Not used by the 8086; reserved for higher processors like 80286, 80386 etc. (27 interrupts) |
| 3 | Type 32 to Type 255 | Available for the user → called user-defined interrupts; can be either H/W interrupts activated through the INTR line or S/W interrupts (224 interrupts) |

## 8.5 Dedicated Interrupts — Types 0 to 4 (MUST MEMORISE)

| Type | Name | Description |
| --- | --- | --- |
| Type 0 | Divide by Zero Error Interrupt | The quotient is large and cannot fit in AL/AX, or divide by zero. (Slide example: `MOV AL,05H / MOV DL,00H / DIV DL / HLT` → "Attempted to divide by zero" error.) |
| Type 1 | Single step or Trap Interrupt | Used for executing the program in single-step mode by setting the trap flag. |
| Type 2 | Non-Maskable Interrupt (NMI) | Used for executing the ISR of the NMI pin (positive edge signal). NMI cannot be masked by software. |
| Type 3 | One-byte INT instruction interrupt | Used for providing break points in the program. |
| Type 4 | Overflow Interrupt | Used to handle any overflow error after signed arithmetic. |

# 9. MOST LIKELY EXAM QUESTIONS (ARCHITECTURE)

| Question | Answer skeleton (write these tables/points) |
| --- | --- |
| Draw and explain the architecture of 8086. | Split the diagram into BIU (CS/DS/SS/ES segment registers, IP, dedicated 20-bit adder, 6-byte FIFO queue) and EU (16-bit ALU, AX/BX/CX/DX, SP/BP, SI/DI, flag register). Explain pipelining: BIU fetches the next instruction while the EU executes the current one. |
| Explain the pin diagram of 8086 / distinguish minimum-mode and maximum-mode pins. | AD0–AD15, A16/S3–A19/S6, BHE/S7, MN/MX, RD, READY, RESET, CLK, INTR, NMI, TEST as common. Min mode: ALE, DEN, DT/R, M/IO, WR, INTA, HOLD, HLDA. Max mode: S0–S2, QS0–QS1, LOCK, RQ/GT0–RQ/GT1. Add the S2S1S0 machine-cycle table and the QS1QS0 queue table. |
| What are the functions of ALE, DEN and DT/R? | ALE → latch the address (demultiplexing via 8282; valid address indication). DEN → output enable for the 8286 transceiver. DT/R → direction of data flow through the transceiver. |
| Difference between minimum and maximum mode of 8086. | Use the §4.4 comparison table: MN/MX value, coprocessor use, 8288 requirement, signals generated. |
| Explain the register organisation of 8086. | 4 groups + special functions table of §6 (AX accumulator, BX base, CX count, DX data, SP/BP pointers, SI/DI index, IP, CS/DS/SS/ES, flag register). |
| Explain the flag register of 8086. | Bit positions: CF (bit 0), PF (2), AF (4), ZF (6), SF (7), TF (8), IF (9), DF (10), OF (11) with the condition that sets each. |
| How does the 8086 compute the physical address? | PA = (Segment × 16₁₀) + Offset; worked example 89AB:F012 = 98AC2H; explain the 20-bit vs 16-bit register problem and the paragraph concept. |
| Explain memory segmentation in 8086. | 1 MB split into segments of up to 64 KB; 4 active segments (256 KB) at a time; CS/DS/SS/ES with their offsets; memory map example. |
| What is pipelining? How is it implemented in 8086? | Fetch and execute simultaneously; BIU + EU split; 6-byte instruction queue; benefit = overlapping fetch with execution. |
| What is the instruction queue? | 6-byte FIFO in the BIU that pre-fetches instruction bytes ahead of time to overlap fetch with execution. |
| List the sources/types of interrupts in 8086. | Hardware (INTR maskable, NMI non-maskable), software (INT n), error conditions (divide by zero, overflow); classification chart. |
| Explain the interrupt processing steps of the 8086. | Push flags → clear IF → reset TF → push CS → push IP → indirect far jump to ISR (§8.2). |
| What is the IVT? How is the interrupt vector address calculated? | 00000H–003FFH, 1 KB, 256 vectors × 4 bytes; Offset = Type × 4; lower 2 bytes = IP, upper 2 bytes = CS; example INT 02H → 00008H. |
| Explain the dedicated interrupts (Types 0–4). | Table of §8.5 with the four examples. |
| Why can't NMI be masked? Where is it used? | It is a positive-edge, non-maskable input; uses: power failure, critical response times, non-recoverable hardware errors, watchdog, memory parity errors. |

# 10. LAST-MINUTE CRAM SHEET (ARCHITECTURE)

Numbers to remember:

| Item | Value |
| --- | --- |
| 8086 released / word size | 1978 / 16-bit |
| Transistors | ≈29,000 |
| Package / supply / clock duty cycle | 40-pin DIP / +5 V / 33 % (asymmetric) |
| Address bus / memory | 20-bit / 1 MB (2²⁰) |
| Data bus | 16-bit |
| Segment size / active segments | 64 KB / 4 segments = 256 KB |
| Instruction queue | 6 bytes FIFO |
| Physical address formula | (Seg × 16₁₀) + Offset |
| IVT location & size | 00000H–003FFH, 1 KB |
| Interrupt vector size / count | 4 bytes / 256 vectors |
| Vector address | Type number × 4 |
| Interrupt groups | 5 dedicated (0–4), 27 reserved (5–31), 224 available (32–255) |
| RESET must be high for | at least 4 clock cycles |

Flags: CF bit 0 · PF 2 · AF 4 · ZF 6 · SF 7 · TF 8 · IF 9 · DF 10 · OF 11.

Modes: MN/MX = 1 → Minimum (no coprocessor); MN/MX = 0 → Maximum (multiprocessor, needs 8288).

Chips: 8284 = clock generator · 8282 = latch (address demux with ALE) · 8286 = transceiver (data bus) · 8288 = bus controller (max mode).

Registers: AX/BX/CX/DX (A=accumulator, B=base, C=count, D=data) · SP/BP · SI/DI · IP · CS/DS/SS/ES.

Two things you must never confuse: MN/MX = 1 is minimum, and INTR is maskable while NMI is not.

Continue with Part 2 (Addressing Modes) and Part 3 (Instruction Set) — they carry the remaining exam questions of Module 2.