# MPMC — Module 2 · PART 2: 8086 ADDRESSING MODES — EXAM-READY NOTES

### Topic: Memory access fundamentals (EA · BA · MA, Segment:Offset, physical address) + all 12 addressing modes

Source: Module 2 lecture slides (ECE3004 – Microprocessors and Microcontrollers).

Module 2 is split into 3 files — use them together, nothing is left out:

•  Part 1 → MPMC_Module2_Part1_Architecture.md — generations, pins, BIU/EU, registers, flags, physical address, interrupts

•  Part 2 (this file) → all 12 addressing modes with worked examples

•  Part 3 → MPMC_Module2_Part3_Instruction_Set.md — instruction set, assembler directives, procedures/macros, programs

•  (All-in-one master copy: MPMC_Module2_Exam_Notes.md)

## CONTENTS OF THIS FILE

1.  What is an addressing mode

2.  Memory access fundamentals — EA, BA, MA and physical address calculation

3.  Which registers/combinations are allowed inside [ ]

4.  Group I — addressing modes for register and immediate data

5.  Group II — addressing modes for memory data

6.  Group III — addressing modes for I/O ports

7.  Group IV — relative addressing mode

8.  Group V — implied addressing mode

9.  Master summary table of all 12 modes

10.  Most likely exam questions (addressing modes)

11.  Last-minute revision cram sheet (addressing modes)

# 1. WHAT IS AN ADDRESSING MODE?

Definition (slide wording): Every instruction of a program has to operate on a data. The different ways in which a source operand is denoted in an instruction are known as addressing modes.

The 12 addressing modes, arranged in the slide's 5 groups:

| Group | Addressing modes |
| --- | --- |
| Group I — for register and immediate data | 1. Register Addressing, 2. Immediate Addressing |
| Group II — for memory data | 3. Direct, 4. Register Indirect, 5. Based, 6. Indexed, 7. Based Index, 8. String |
| Group III — for I/O ports | 9. Direct I/O port, 10. Indirect I/O port |
| Group IV — relative | 11. Relative Addressing |
| Group V — implied | 12. Implied Addressing |

# 2. MEMORY ACCESS FUNDAMENTALS

## 2.1 The problem

•  The 8086 has 20 address lines → can address up to 2²⁰ = 1 M bytes of memory.

•  However, the largest register is only 16 bits.

•  Therefore the physical address has to be calculated.

## 2.2 Definitions

•  Physical Address: the actual address of a byte in memory, i.e. the value which goes out onto the address bus (20 bits).

•  Memory address is represented in the form Seg : Offset — e.g. 89AB:F012.

•  16 bytes of contiguous memory = one paragraph (hence "paragraph to byte" = × 10H in hex).

## 2.3 The calculation (slide method)

Each time the processor wants to access memory, it takes the contents of a segment register, shifts it one hexadecimal place to the left (same as multiplying by 16₁₀), then adds the required offset to form the 20-bit address.

Physical Address = (Segment register × 16₁₀) + Offset

Worked example — 89AB:F012

Segment 89AB → 89AB0 (paragraph to byte → 89AB × 10H = 89AB0) Offset F012 → F012 (offset is already in byte units) ----- Physical address 98AC2 H (the absolute address)

The adder block diagram (slide figure): a 20-bit adder takes the 16-bit segment register on one side (with the low 4 bits as 0000) and the 16-bit offset value on the other side to produce the 20-bit physical address.

## 2.4 The three quantities used throughout this file

| Symbol | Stands for | Formula |
| --- | --- | --- |
| EA | Effective Address (the offset part) | Depends on the addressing mode |
| BA | Base Address | BA = (Segment register) × 16₁₀ |
| MA | Memory Address | MA = BA + EA |

For a word operation, the address pair is written (MA ; MA + 1) — the low byte at MA and the high byte at MA+1.

## 2.5 Default segment registers for each offset register

| Offset register used | Default segment register |
| --- | --- |
| BX, SI, DI, 16-bit displacement (disp16) | DS |
| BP | SS |
| SP (stack instructions) | SS |
| IP (instruction fetch) | CS |
| String destination (DI) | ES |

In the slides the rule is stated as: "Content of the DS register is used for base address calculation" for register indirect; BX with DS, BP with SS; string source = DS:SI, string destination = ES:DI.

# 3. ALLOWED REGISTER COMBINATIONS INSIDE [ ]

To access memory we use these four registers: BX, SI, DI, BP. Combining these registers inside [ ] symbols gives different memory locations (Effective Address, EA).

Supported combinations (exact table from the slides):

| With no displacement | With 8-bit displacement (d8) | With 16-bit displacement (d16) |
| --- | --- | --- |
| [BX + SI] | [BX + SI + d8] | [BX + SI + d16] |
| [BX + DI] | [BX + DI + d8] | [BX + DI + d16] |
| [BP + SI] | [BP + SI + d8] | [BP + SI + d16] |
| [BP + DI] | [BP + DI + d8] | [BP + DI + d16] |
| [SI] | [SI + d8] | [SI + d16] |
| [DI] | [DI + d8] | [DI + d16] |
| [BX] | [BX + d8] | [BX + d16] |
| [BP] | [BP + d8] | [BP + d16] |
| — | — | d16 (variable offset only) |

Rule: Base = BX or BP (BP → SS); Index = SI or DI (both → DS). You may use one base, one index, and one displacement — nothing else.

# 4. GROUP I — ADDRESSING MODES FOR REGISTER AND IMMEDIATE DATA

## 4.1 Register Addressing

The instruction specifies the name of the register which holds the data to be operated upon.

MOV CL, DH (CL) ← (DH) ADD AX, BX (AX) ← (AX) + (BX)

•  The content of the 8-bit register DH (source) is only copied to another 8-bit register CL (destination).

•  The content of the source will NOT be cleared after the execution of this instruction.

## 4.2 Immediate Addressing

An 8-bit or 16-bit data is specified as part of the instruction.

MOV DL, 08H (DL) ← 08H SUB AX, 0A9FH (AX) ← (AX) – 0A9FH

The 8-bit data (08H) given in the instruction is moved to DL; the 16-bit data (0A9FH) is subtracted from AX.

# 5. GROUP II — ADDRESSING MODES FOR MEMORY DATA

## 5.1 Direct Addressing

•  The effective address of the memory location at which the data operand is stored is given in the instruction.

•  The effective address is just a 16-bit number written directly in the instruction.

•  This mode is called direct because the displacement of the operand from the segment base is specified directly in the instruction.

MOV [2000H], 02H MOV [0234H], 0234H MOV BX, [1354H] MOV [1234H], BX MOV BL, [1354H] MOV [0400H], CL

The square brackets around 1354H denote the contents of the memory location. When executed, MOV BX, [1354H] copies the contents of memory location 1354H into the BX register.

## 5.2 Register Indirect Addressing

•  The name of the register which holds the effective address (EA) is specified in the instruction.

•  Registers used to hold EA: BX, BP, DI and SI.

•  Content of the DS register is used for base address calculation.

MOV CX, [BX] MOV [BX], AL MOV [BP], 52H MOV [SI], 1234H

Operations for `MOV CX, [BX]`:

EA = (BX) BA = (DS) × 16₁₀ MA = BA + EA (CX) ← (MA) or, (CL) ← (MA) (CH) ← (MA + 1)

Note (slide): Register/memory enclosed in brackets refers to the content of the register/memory.

## 5.3 Based Addressing

•  BX or BP is used to hold the base value for the effective address, and a signed 8-bit or unsigned 16-bit displacement is specified in the instruction.

•  In case of an 8-bit displacement, it is sign-extended to 16 bits before adding to the base value.

•  When BX holds the base value of EA → the 20-bit physical address is calculated from BX and DS.

•  When BP holds the base value of EA → BP and SS are used.

MOV AX, [BX + 08H] MOV [BX + 1234H], AX MOV [BP + 21H], 9878H

Operations for `MOV AX, [BX + 08H]`:

0008H ← 08H (sign extended) EA = (BX) + 0008H BA = (DS) × 16₁₀ MA = BA + EA (AX) ← (MA) or, (AL) ← (MA) (AH) ← (MA + 1)

## 5.4 Indexed Addressing

•  SI or DI register is used to hold an index value for memory data and a signed 8-bit or unsigned 16-bit displacement is specified in the instruction.

•  The displacement is added to the index value in SI or DI to obtain the EA.

•  In case of an 8-bit displacement, it is sign-extended to 16 bits before adding.

MOV CX, [SI + 0A2H]

FFA2H ← A2H (sign extended) EA = (SI) + FFA2H BA = (DS) × 16₁₀ MA = BA + EA (CX) ← (MA) or, (CL) ← (MA) (CH) ← (MA + 1)

## 5.5 Based Index Addressing

•  The effective address is computed from the sum of a base register (BX or BP), an index register (SI or DI) and a displacement.

MOV DX, [BX + SI + 0AH]

000AH ← 0AH (sign extended) EA = (BX) + (SI) + 000AH BA = (DS) × 16₁₀ MA = BA + EA (DX) ← (MA) or, (DL) ← (MA) (DH) ← (MA + 1)

## 5.6 String Addressing

•  Employed in string operations to operate on string data.

•  The effective address (EA) of source data is stored in the SI register and the EA of destination is stored in the DI register.

•  The segment register for calculating the base address of source data is DS and that of the destination data is ES.

MOVS BYTE

Operations:

Calculation of source memory location: EA = (SI) BA = (DS) × 16₁₀ MA = BA + EA Calculation of destination memory location: EAE = (DI) BAE = (ES) × 16₁₀ MAE = BAE + EAE (MAE) ← (MA) If DF = 1 → (SI) ← (SI) – 1 and (DI) ← (DI) – 1 If DF = 0 → (SI) ← (SI) + 1 and (DI) ← (DI) + 1

Note: EAE = Effective address of the Extra segment register. (For word strings the increment/decrement is 2 instead of 1 — see Part 3, string instructions.)

# 6. GROUP III — ADDRESSING MODES FOR I/O PORTS

These addressing modes are used to access data from standard I/O-mapped devices or ports.

## 6.1 Direct I/O Port Addressing

In direct port addressing mode, an 8-bit port address is directly specified in the instruction.

IN AL, [09H]

PORTaddr = 09H (AL) ← (PORT) ; content of the port with address 09H is moved to AL

## 6.2 Indirect I/O Port Addressing

In indirect port addressing mode, the instruction specifies the name of the register which holds the port address. In the 8086, the 16-bit port address is stored in the DX register.

OUT [DX], AX

PORTaddr = (DX) (PORT) ← (AX) ; content of AX is moved to the port whose address is in DX

Compare with memory indirect: memory indirect can use BX/BP/SI/DI, but I/O indirect uses only DX.

# 7. GROUP IV — RELATIVE ADDRESSING MODE

•  In this addressing mode, the effective address of a program instruction is specified relative to the Instruction Pointer (IP) by an 8-bit signed displacement.

JZ 0AH

Operations:

000AH ← 0AH (sign extend) If ZF = 1, then EA = (IP) + 000AH BA = (CS) × 16₁₀ MA = BA + EA → the program control jumps to the new address calculated above If ZF = 0, then the next instruction of the program is executed

Used by the whole family of conditional jumps and LOOP instructions — see Part 3, control transfer instructions.

# 8. GROUP V — IMPLIED ADDRESSING MODE

•  Instructions using this mode have no operands.

•  The instruction itself will specify the data to be operated on by the instruction.

CLC ; this clears the carry flag to zero (CF ← 0)

Same family: STC, CMC, CLD, STD, CLI, STI, NOP, HLT.

# 9. MASTER SUMMARY TABLE OF ALL 12 MODES

Use this to answer "list and explain the addressing modes with examples":

| # | Mode | Group | Example | Where the operand / its address comes from |
| --- | --- | --- | --- | --- |
| 1 | Register | I | MOV CL, DH | Inside a register named in the instruction |
| 2 | Immediate | I | MOV DL, 08H | Data is part of the instruction itself |
| 3 | Direct | II | MOV BX, [1354H] | 16-bit EA written directly in the instruction |
| 4 | Register Indirect | II | MOV CX, [BX] | EA is held in BX/BP/SI/DI |
| 5 | Based | II | MOV AX, [BX + 08H] | (BX/BP) + disp (8-bit sign-extended / 16-bit) |
| 6 | Indexed | II | MOV CX, [SI + 0A2H] | (SI/DI) + disp |
| 7 | Based Index | II | MOV DX, [BX + SI + 0AH] | (BX/BP) + (SI/DI) + disp |
| 8 | String | II | MOVS BYTE | Source DS:SI, destination ES:DI, auto ±1/±2 by DF |
| 9 | Direct I/O port | III | IN AL, [09H] | 8-bit port address in the instruction |
| 10 | Indirect I/O port | III | OUT [DX], AX | 16-bit port address in DX |
| 11 | Relative | IV | JZ 0AH | (IP) + 8-bit signed displacement, base CS |
| 12 | Implied | V | CLC | Operand implied by the instruction itself |

One-line classification trick: for memory modes, remember "Direct = address in instruction; Indirect = address in register; Based = BX/BP + disp; Indexed = SI/DI + disp; Based-Index = both + disp; String = SI/DI with DS/ES."

Sign-extension rule (very commonly asked): an 8-bit displacement is sign-extended to 16 bits before adding (so [BX + 08H] adds 0008H, while [SI + 0A2H] adds FFA2H because A2H has its MSB set).

# 10. MOST LIKELY EXAM QUESTIONS (ADDRESSING MODES)

| Question | Answer skeleton |
| --- | --- |
| Define addressing mode. List and explain the addressing modes of 8086 with examples. | Start with the definition, then the 5-group table, then explain each of the 12 modes with the slide example and its EA/BA/MA calculation. Finish with the master summary table of §9. |
| Explain the addressing modes for memory data (Group II). | Direct, Register indirect, Based, Indexed, Based-index, String — each with the worked EA = …, BA = (DS) × 16₁₀, MA = BA + EA chain. |
| Explain the addressing modes of the 8086 used for I/O ports. | Direct port (8-bit port address in instruction, IN AL,[09H]) and Indirect port (16-bit port address in DX, OUT [DX],AX). |
| How does the 8086 generate a 20-bit physical address from two 16-bit values? | Segment register shifted left one hex digit (×16₁₀) + offset, using the 20-bit adder; worked example 89AB:F012 = 98AC2H; represent as Seg:Offset. |
| Which registers can be used inside `[ ]` and in which combinations? | BX/BP (base) and SI/DI (index) only, optionally with d8/d16 — reproduce the combination table of §3. |
| What is the default segment register for BX / BP / SP / SI / DI / IP? | DS for BX, SI, DI and disp; SS for BP and SP; CS for IP; ES for the string destination (DI). |
| Give an example showing sign extension of an 8-bit displacement. | MOV CX, [SI + 0A2H] → FFA2H sign-extended, EA = (SI) + FFA2H. |
| Differentiate direct and register-indirect addressing. | Direct: 16-bit EA written in the instruction (e.g. MOV BX,[1354H]). Register indirect: EA held in a register (BX/BP/SI/DI), e.g. MOV CX,[BX]. |
| Which addressing mode do conditional jumps use? | Relative addressing — EA = (IP) + 8-bit signed displacement, base address from CS; control transfers only if the flag condition is true. |
| Write a note on implied addressing mode. | No operands in the instruction; the instruction itself specifies the data (e.g. CLC clears CF). |

# 11. LAST-MINUTE CRAM SHEET (ADDRESSING MODES)

The 3 formulas to write first in any answer:

EA = Effective Address (offset part) BA = (Segment register) × 16₁₀ MA = BA + EA [ for a word: (MA ; MA + 1) ]

Physical address:

Physical Address = (Segment × 16₁₀) + Offset e.g. 89AB:F012 → 98AC2H

Registers allowed in `[ ]`: BX, BP, SI, DI (+ d8 / d16 displacement). Base = BX or BP · Index = SI or DI · Displacement = 8-bit (sign-extended) or 16-bit.

Defaults: BX/SI/DI/disp → DS · BP and SP → SS · IP → CS · String destination DI → ES.

Memory modes checklist: Direct (address in instruction) → Register indirect (address in register) → Based (BX/BP + disp) → Indexed (SI/DI + disp) → Based-index (both + disp) → String (SI/DI, DS/ES).

I/O: Direct = 8-bit port address in the instruction; Indirect = 16-bit port address in DX (only DX).

Brackets `[ ]` mean "contents of" — never the address itself.

Continue with Part 1 (Architecture) and Part 3 (Instruction Set) — they carry the remaining exam questions of Module 2.