# MPMC — Module 2 · PART 3: 8086 INSTRUCTION SET, DIRECTIVES & PROGRAMMING — EXAM-READY NOTES

### Topic: All 6 instruction groups · String instructions · Branch & loop instructions · Assembler directives & operators · Procedures, CALL/RET, macros · Modular programming · Example programs

Source: Module 2 lecture slides (ECE3004 – Microprocessors and Microcontrollers).

Module 2 is split into 3 files — use them together, nothing is left out:

•  Part 1 → MPMC_Module2_Part1_Architecture.md — generations, pins, BIU/EU, registers, flags, physical address, interrupts

•  Part 2 → MPMC_Module2_Part2_Addressing_Modes.md — all 12 addressing modes

•  Part 3 (this file) → instruction set, assembler directives, procedures/macros, programs

•  (All-in-one master copy: MPMC_Module2_Exam_Notes.md)

## CONTENTS OF THIS FILE

1.  The 6 types of instructions — overview & notation

2.  Data transfer instructions

3.  Arithmetic instructions

4.  Logical instructions (including shifts and rotates)

5.  String manipulation instructions

6.  Processor control instructions

7.  Control transfer instructions (jumps, branches, loops)

8.  Assembler directives and assembler operators

9.  Procedures, CALL/RET, macros and modular programming

10.  Example programs

11.  Most likely exam questions (instruction set)

12.  Last-minute revision cram sheet (instruction set)

# 1. THE 6 TYPES OF INSTRUCTIONS — OVERVIEW

The 8086 supports 6 types of instructions:

1.  Data Transfer Instructions

2.  Arithmetic Instructions

3.  Logical Instructions

4.  String Manipulation Instructions

5.  Processor Control Instructions

6.  Control Transfer Instructions

Notation used in this file (same as the slides):

•  ( ) denotes contents of

•  ← denotes transfer / assignment

•  reg2 = destination, reg1 = source (as written in the slides)

•  MA = memory address, MAE = memory address in the Extra segment, BA = base address, EA = effective address

•  d8 / d16 = 8-bit / 16-bit displacement

# 2. DATA TRANSFER INSTRUCTIONS

Definition (slide): Instructions that are used to transfer data/address into registers, memory locations and I/O ports. Generally involve two operands — source and destination — of the same size. The source can be a register, a memory location or immediate data; the destination is a register or a memory location. The size should be either a byte or a word. An 8-bit data can only be moved to an 8-bit register/memory and a 16-bit data to a 16-bit register/memory.

Mnemonics: MOV, XCHG, PUSH, POP, IN, OUT

| Instruction | Operation |
| --- | --- |
| MOV reg2, reg1 | (reg2) ← (reg1) |
| MOV mem, reg1 | (mem) ← (reg1) |
| MOV reg2, mem | (reg2) ← (mem) |
| MOV reg, data | (reg) ← data |
| MOV mem, data | (mem) ← data |
| XCHG reg2, reg1 | (reg2) ↔ (reg1) |
| XCHG mem, reg1 | (mem) ↔ (reg1) |
| PUSH reg16 | (SP) ← (SP) – 2 ; MA_S = (SS) × 16₁₀ + SP ; (MA_S ; MA_S + 1) ← (reg16) |
| PUSH mem | (SP) ← (SP) – 2 ; MA_S = (SS) × 16₁₀ + SP ; (MA_S ; MA_S + 1) ← (mem) |
| POP reg16 | MA_S = (SS) × 16₁₀ + SP ; (reg16) ← (MA_S ; MA_S + 1) ; (SP) ← (SP) + 2 |
| POP mem | MA_S = (SS) × 16₁₀ + SP ; (mem) ← (MA_S ; MA_S + 1) ; (SP) ← (SP) + 2 |
| IN AL, [DX] | PORTaddr = (DX) ; (AL) ← (PORT) |
| IN AX, [DX] | PORTaddr = (DX) ; (AX) ← (PORT) |
| IN AL, addr8 | (AL) ← (addr8) |
| IN AX, addr8 | (AX) ← (addr8) |
| OUT [DX], AL | PORTaddr = (DX) ; (PORT) ← (AL) |
| OUT [DX], AX | PORTaddr = (DX) ; (PORT) ← (AX) |
| OUT addr8, AL | (addr8) ← (AL) |
| OUT addr8, AX | (addr8) ← (AX) |

Points to remember (very common short questions):

•  MOV cannot transfer memory → memory, segment register → segment register, or an immediate → segment register.

•  XCHG swaps the two operands (no immediate operand allowed).

•  PUSH decrements SP by 2 then stores; POP loads then increments SP by 2 (stack grows downwards).

•  I/O indirect addressing uses only DX for the 16-bit port address.

# 3. ARITHMETIC INSTRUCTIONS

Mnemonics: ADD, ADC, SUB, SBB, INC, DEC, MUL, DIV, IMUL, IDIV, CMP

## 3.1 ADD / ADC

| Instruction | Operation |
| --- | --- |
| ADD reg2, reg1 | (reg2) ← (reg1) + (reg2) |
| ADD reg2, mem | (reg2) ← (reg2) + (mem) |
| ADD mem, reg1 | (mem) ← (mem) + (reg1) |
| ADD reg, data | (reg) ← (reg) + data |
| ADD mem, data | (mem) ← (mem) + data |
| ADD AL, data8 | (AL) ← (AL) + data8 |
| ADD AX, data16 | (AX) ← (AX) + data16 |
| ADC reg2, reg1 | (reg2) ← (reg1) + (reg2) + CF |
| ADC reg2, mem | (reg2) ← (reg2) + (mem) + CF |
| ADC mem, reg1 | (mem) ← (mem) + (reg1) + CF |
| ADC reg, data | (reg) ← (reg) + data + CF |
| ADC mem, data | (mem) ← (mem) + data + CF |
| ADC AL, data8 | (AL) ← (AL) + data8 + CF |
| ADC AX, data16 | (AX) ← (AX) + data16 + CF |

ADC = ADD with carry. Used for multi-byte (multi-word) addition.

## 3.2 SUB / SBB

| Instruction | Operation |
| --- | --- |
| SUB reg2, reg1 | (reg2) ← (reg1) – (reg2) |
| SUB reg2, mem | (reg2) ← (reg2) – (mem) |
| SUB mem, reg1 | (mem) ← (mem) – (reg1) |
| SUB reg, data | (reg) ← (reg) – data |
| SUB mem, data | (mem) ← (mem) – data |
| SUB AL, data8 | (AL) ← (AL) – data8 |
| SUB AX, data16 | (AX) ← (AX) – data16 |
| SBB reg2, reg1 | (reg2) ← (reg1) – (reg2) – CF |
| SBB reg2, mem | (reg2) ← (reg2) – (mem) – CF |
| SBB mem, reg1 | (mem) ← (mem) – (reg1) – CF |
| SBB reg, data | (reg) ← (reg) – data – CF |
| SBB mem, data | (mem) ← (mem) – data – CF |
| SBB AL, data8 | (AL) ← (AL) – data8 – CF |
| SBB AX, data16 | (AX) ← (AX) – data16 – CF |

SBB = SUB with borrow. Used for multi-byte subtraction.

Note: as written in the slides, SUB reg2, reg1 is defined as (reg1) – (reg2). Reproduce the slide's convention in your exam answers (the confusion arises because the slides name the destination reg2).

## 3.3 INC / DEC

| Instruction | Operation |
| --- | --- |
| INC reg8 | (reg8) ← (reg8) + 1 |
| INC reg16 | (reg16) ← (reg16) + 1 |
| INC mem | (mem) ← (mem) + 1 |
| DEC reg8 | (reg8) ← (reg8) – 1 |
| DEC reg16 | (reg16) ← (reg16) – 1 |
| DEC mem | (mem) ← (mem) – 1 |

INC/DEC do NOT affect the carry flag CF (they do affect ZF, SF, PF, AF, OF).

## 3.4 MUL / IMUL — unsigned and signed multiplication

| Instruction | Byte operation | Word operation |
| --- | --- | --- |
| MUL reg | (AX) ← (AL) × (reg8) | (DX)(AX) ← (AX) × (reg16) |
| MUL mem | (AX) ← (AL) × (mem8) | (DX)(AX) ← (AX) × (mem16) |
| IMUL reg | (AX) ← (AL) × (reg8) | (DX)(AX) ← (AX) × (reg16) |
| IMUL mem | (AX) ← (AX) × (mem8) (as printed in the slides) | (DX)(AX) ← (AX) × (mem16) |

Key rule (memorise the register pairs):

•  Byte × Byte → 16-bit product in AX (AL is the implicit multiplicand).

•  Word × Word → 32-bit product in the DX:AX pair (AX is the implicit multiplicand; DX holds the high word).

•  IMUL = signed multiply — the flags CF and OF indicate whether the upper half is significant (useful for exam one-liners).

## 3.5 DIV / IDIV — unsigned and signed division

| Instruction | 16 ÷ 8 (word ÷ byte) | 32 ÷ 16 (dword ÷ word) |
| --- | --- | --- |
| DIV reg | (AL) ← (AX) ÷ (reg8) → quotient · (AH) ← (AX) MOD (reg8) → remainder | (AX) ← (DX)(AX) ÷ (reg16) → quotient · (DX) ← (DX)(AX) MOD (reg16) → remainder |
| DIV mem | (AL) ← (AX) ÷ (mem8) · (AH) ← (AX) MOD (mem8) | (AX) ← (DX)(AX) ÷ (mem16) · (DX) ← (DX)(AX) MOD (mem16) |
| IDIV reg | (AL) ← (AX) ÷ (reg8) · (AH) ← (AX) MOD (reg8) | (AX) ← (DX)(AX) ÷ (reg16) · (DX) ← (DX)(AX) MOD (reg16) |
| IDIV mem | (AL) ← (AX) ÷ (mem8) · (AH) ← (AX) MOD (mem8) | (AX) ← (DX)(AX) ÷ (mem16) · (DX) ← (DX)(AX) MOD (mem16) |

CRITICAL EXAM POINT:

•  Word ÷ Byte: dividend AX → quotient in AL, remainder in AH.

•  Dword ÷ Word: dividend DX:AX → quotient in AX, remainder in DX.

•  The remainder always takes the size of the divisor (i.e. AH for a byte divisor, DX for a word divisor).

•  IDIV = signed divide, same register conventions.

•  If the quotient is too large to fit (or the divisor is 0) → Type-0 divide-by-zero interrupt is generated.

## 3.6 CMP (Compare)

| Instruction | Operation |
| --- | --- |
| CMP reg2, reg1 | Modify flags ← (reg2) – (reg1) |
| CMP reg2, mem | Modify flags ← (reg2) – (mem) |
| CMP mem, reg1 | Modify flags ← (mem) – (reg1) |
| CMP reg, data | Modify flags ← (reg) – data |
| CMP mem, data | Modify flags ← (mem) – data |
| CMP AL, data8 | Modify flags ← (AL) – data8 |
| CMP AX, data16 | Modify flags ← (AX) – data16 |

Flag table (memorise — it is used for every conditional jump):

| Condition | CF | ZF | SF |
| --- | --- | --- | --- |
| (reg2) > (reg1) | 0 | 0 | 0 |
| (reg2) < (reg1) | 1 | 0 | 1 |
| (reg2) = (reg1) | 0 | 1 | 0 |

CMP modifies flags only — it does NOT change any operand. Always followed by a conditional jump.

# 4. LOGICAL INSTRUCTIONS (including shifts and rotates)

Mnemonics: AND, OR, XOR, NOT, TEST, SHR, SHL/SAL, RCR, RCL, ROR, ROL

## 4.1 AND

| Instruction | Operation |
| --- | --- |
| AND reg2, reg1 | (reg2) ← (reg2) & (reg1) |
| AND reg2, mem | (reg2) ← (reg2) & (mem) |
| AND mem, reg1 | (mem) ← (mem) & (reg1) |
| AND reg, data | (reg) ← (reg) & data |
| AND mem, data | (mem) ← (mem) & data |
| AND AL, data8 | (AL) ← (AL) & data8 |
| AND AX, data16 | (AX) ← (AX) & data16 |

Uses: masking/clearing bits, checking whether a bit is 0 or 1.

## 4.2 OR

| Instruction | Operation |
| --- | --- |
| OR reg2, reg1 | (reg2) ← (reg2) ¦ (reg1) |
| OR reg2, mem | (reg2) ← (reg2) ¦ (mem) |
| OR mem, reg1 | (mem) ← (mem) ¦ (reg1) |
| OR reg, data | (reg) ← (reg) ¦ data |
| OR mem, data | (mem) ← (mem) ¦ data |
| OR AL, data8 | (AL) ← (AL) ¦ data8 |
| OR AX, data16 | (AX) ← (AX) ¦ data16 |

Use: setting bits.

## 4.3 XOR

| Instruction | Operation |
| --- | --- |
| XOR reg2, reg1 | (reg2) ← (reg2) ^ (reg1) |
| XOR reg2, mem | (reg2) ← (reg2) ^ (mem) |
| XOR mem, reg1 | (mem) ← (mem) ^ (reg1) |
| XOR reg, data | (reg) ← (reg) ^ data |
| XOR mem, data | (mem) ← (mem) ^ data |
| XOR AL, data8 | (AL) ← (AL) ^ data8 |
| XOR AX, data16 | (AX) ← (AX) ^ data16 |

Uses: toggling/complementing selected bits; `XOR reg, reg` clears a register (shortcut).

## 4.4 TEST

| Instruction | Operation |
| --- | --- |
| TEST reg2, reg1 | Modify flags ← (reg2) & (reg1) (operands unchanged) |
| TEST reg2, mem | Modify flags ← (reg2) & (mem) |
| TEST mem, reg1 | Modify flags ← (mem) & (reg1) |
| TEST reg, data | Modify flags ← (reg) & data |
| TEST mem, data | Modify flags ← (mem) & data |
| TEST AL, data8 | Modify flags ← (AL) & data8 |
| TEST AX, data16 | Modify flags ← (AX) & data16 |

TEST vs AND (slide wording): The TEST instruction works the same as the AND operation, but unlike the AND instruction it does not change the first operand. So, if we need to check whether a number in a register is even or odd, we can do this using the TEST instruction without changing the original number.

## 4.5 SHR — logical shift right

Instruction forms

SHR reg, 1 or SHR reg, CL

SHR mem, 1 or SHR mem, CL

Operation (slide formula):

CF ← B_LSD ; B_n ← B_n+1 ; B_MSD ← 0

The MSB is filled with 0, the LSB shifts into CF, and every bit moves one position right. (The slides show the bit chain B7 … B0 → CF for reg8/mem8 and B15 … B0 → CF for reg16/mem16.)

## 4.6 SHL / SAL — logical shift left (arithmetic shift left)

Instruction forms

SHL reg, 1 or SHL reg, CL (same for SAL)

SHL mem, 1 or SHL mem, CL

Operation (slide formula):

CF ← B_MSD ; B_n+1 ← B_n ; B_LSD ← 0

The LSB is filled with 0, the MSB shifts into CF, and every bit moves one position left.

Doubling trick: SHL by 1 = multiply by 2; SHR by 1 = divide by 2 (unsigned).

Count rule: the count must be either `1` (immediate) or `CL` — no other register is allowed.

## 4.7 RCR — rotate through carry right

Instruction forms

RCR reg, 1 or RCR reg, CL

RCR mem, 1 or RCR mem, CL

Operation (slide formula):

B_n ← B_n–1 ; B_MSD ← CF ; CF ← B_LSD

The bits rotate right through the carry flag: the LSB goes into CF, and CF goes into the MSB. Nothing is lost — the carry is part of the rotation loop.

## 4.8 RCL — rotate through carry left

Complement of RCR: the bits rotate left through CF — MSB → CF, CF → LSB.

## 4.9 ROL — rotate left

Instruction forms

ROL reg, 1 or ROL reg, CL

ROL mem, 1 or ROL mem, CL

Operation (slide formula):

B_n–1 ← B_n ; CF ← B_MSD ; B_LSD ← B_MSD

The MSB is copied to the LSB and also to CF — a plain circular rotation (no carry in the loop).

## 4.10 ROR — rotate right

Complement of ROL: LSB → MSB and LSB → CF.

Shift/rotate quick table (write this in the exam):

| Instruction | Direction | MSB/LSB filled with | CF gets |
| --- | --- | --- | --- |
| SHR | right | 0 into MSB | old LSB |
| SHL/SAL | left | 0 into LSB | old MSB |
| RCR | right | CF into MSB | old LSB |
| RCL | left | CF into LSB | old MSB |
| ROR | right | old LSB into MSB | old LSB |
| ROL | left | old MSB into LSB | old MSB |

# 5. STRING MANIPULATION INSTRUCTIONS

Facts to quote first:

•  String: a sequence of bytes or words.

•  The 8086 instruction set includes instructions for string movement, comparison, scan, load and store.

•  The REP instruction prefix is used to repeat execution of string instructions.

•  String instructions end with S, SB or SW — S = String, SB = String Byte, SW = String Word.

•  The offset or effective address of the source operand is stored in the SI register and that of the destination operand in the DI register.

•  Depending on the status of DF, the SI and DI registers are automatically updated:

•  DF = 0 ⇒ SI and DI are incremented by 1 for byte and 2 for word.

•  DF = 1 ⇒ SI and DI are decremented by 1 for byte and 2 for word.

Mnemonics: REP, MOVS, CMPS, SCAS, LODS, STOS

## 5.1 REP prefixes

| Prefix | Meaning |
| --- | --- |
| REP | Repeat while CX ≠ 0, and (CX) ← (CX) – 1 |
| REPZ / REPE | While CX ≠ 0 and ZF = 1, repeat the string instruction and (CX) ← (CX) – 1 (Repeat CMPS or SCAS until ZF = 0 — result should be zero for condition true) |
| REPNZ / REPNE | While CX ≠ 0 and ZF = 0, repeat the string instruction and (CX) ← (CX) – 1 (Repeat CMPS or SCAS until ZF = 1 — result should not be zero for condition true) |

Slide note: Always REPZ can be used in association with the string-related operations. Example: `REPZ MOVSB`.

## 5.2 MOVS / MOVSB / MOVSW — move string

MA = (DS) × 16₁₀ + (SI) ; source MAE = (ES) × 16₁₀ + (DI) ; destination (MAE) ← (MA) ; for MOVSB (MAE ; MAE + 1) ← (MA ; MA + 1) ; for MOVSW If DF = 0 → (DI) ← (DI) + 1 ; (SI) ← (SI) + 1 [byte] (DI) ← (DI) + 2 ; (SI) ← (SI) + 2 [word] If DF = 1 → (DI) ← (DI) – 1 ; (SI) ← (SI) – 1 [byte] (DI) ← (DI) – 2 ; (SI) ← (SI) – 2 [word]

## 5.3 CMPS / CMPSB / CMPSW — compare string

Compare two string bytes or string words.

MA = (DS) × 16₁₀ + (SI) MAE = (ES) × 16₁₀ + (DI) Modify flags ← (MA) – (MAE) If (MA) > (MAE) → CF = 0 ; ZF = 0 ; SF = 0 If (MA) < (MAE) → CF = 1 ; ZF = 0 ; SF = 1 If (MA) = (MAE) → CF = 0 ; ZF = 1 ; SF = 0 For byte operation: DF = 0 → (SI),(DI) + 1 ; DF = 1 → (SI),(DI) – 1 For word operation: DF = 0 → (SI),(DI) + 2 ; DF = 1 → (SI),(DI) – 2

## 5.4 SCAS / SCASB / SCASW — scan string

Scan (compare) a string byte or word with the accumulator.

MAE = (ES) × 16₁₀ + (DI) Modify flags ← (AL) – (MAE) [SCASB] Modify flags ← (AX) – (MAE ; MAE+1) [SCASW] If (AL) > (MAE) → CF = 0 ; ZF = 0 ; SF = 0 If (AL) < (MAE) → CF = 1 ; ZF = 0 ; SF = 1 If (AL) = (MAE) → CF = 0 ; ZF = 1 ; SF = 0 If DF = 0 → (DI) ← (DI) + 1 (or +2 for word) If DF = 1 → (DI) ← (DI) – 1 (or –2 for word)

## 5.5 LODS / LODSB / LODSW — load string

Load string byte into AL or string word into AX.

MA = (DS) × 16₁₀ + (SI) (AL) ← (MA) [LODSB] (AX) ← (MA ; MA + 1) [LODSW] If DF = 0 → (SI) ← (SI) + 1 (or +2 for word) If DF = 1 → (SI) ← (SI) – 1 (or –2 for word)

## 5.6 STOS / STOSB / STOSW — store string

Store byte from AL or word from AX into string.

MAE = (ES) × 16₁₀ + (DI) (MAE) ← (AL) [STOSB] (MAE ; MAE + 1) ← (AX) [STOSW] If DF = 0 → (DI) ← (DI) + 1 (or +2 for word) If DF = 1 → (DI) ← (DI) – 1 (or –2 for word)

Memory table of string instructions:

| Instruction | Operands involved | Segment used | Which registers auto-update |
| --- | --- | --- | --- |
| MOVS | source SI → destination DI | DS → ES | SI and DI |
| CMPS | source SI vs destination DI | DS vs ES | SI and DI |
| SCAS | AL/AX vs destination DI | ES | DI only |
| LODS | source SI → AL/AX | DS | SI only |
| STOS | AL/AX → destination DI | ES | DI only |

# 6. PROCESSOR CONTROL INSTRUCTIONS

| Mnemonic | Explanation |
| --- | --- |
| STC | Set carry: CF ← 1 |
| CLC | Clear carry: CF ← 0 |
| CMC | Complement carry: CF ← CF′ |
| STD | Set direction flag: DF ← 1 |
| CLD | Clear direction flag: DF ← 0 |
| STI | Set interrupt enable flag: IF ← 1 |
| CLI | Clear interrupt enable flag: IF ← 0 |
| NOP | No operation |
| HLT | Halt after interrupt is set |
| WAIT | Wait for TEST pin active |
| ESC opcode mem/reg | Used to pass instruction to a coprocessor which shares the address and data bus with the 8086 |
| LOCK | Lock the bus during the next instruction (prevents other bus masters taking the bus) |

Note: STI/CLI affect maskable interrupts only — the NMI can never be masked in software.

# 7. CONTROL TRANSFER INSTRUCTIONS

Facts to quote first:

•  They transfer the control to a specific destination or target instruction.

•  They do NOT affect flags.

## 7.1 Unconditional transfers

| Mnemonic | Explanation |
| --- | --- |
| CALL reg/mem/disp16 | Call subroutine |
| RET | Return from subroutine |
| JMP reg/mem/disp8/disp16 | Unconditional jump |

## 7.2 Conditional branch instructions

•  They check flags.

•  If the conditions are true, the program control is transferred to the new memory location in the same segment by modifying the content of IP.

SIGNED conditional branch instructions:

| Name | Alternate name |
| --- | --- |
| JE disp8 | Jump if equal |
| JZ disp8 | Jump if result is 0 |
| JNE disp8 | Jump if not equal |
| JNZ disp8 | Jump if not zero |
| JG disp8 | Jump if greater |
| JNLE disp8 | Jump if not less or equal |
| JGE disp8 | Jump if greater than or equal |
| JNL disp8 | Jump if not less |
| JL disp8 | Jump if less than |
| JNGE disp8 | Jump if not greater than or equal |
| JLE disp8 | Jump if less than or equal |
| JNG disp8 | Jump if not greater |

UNSIGNED conditional branch instructions:

| Name | Alternate name |
| --- | --- |
| JE disp8 | Jump if equal |
| JZ disp8 | Jump if result is 0 |
| JNE disp8 | Jump if not equal |
| JNZ disp8 | Jump if not zero |
| JA disp8 | Jump if above |
| JNBE disp8 | Jump if not below or equal |
| JAE disp8 | Jump if above or equal |
| JNB disp8 | Jump if not below |
| JB disp8 | Jump if below |
| JNAE disp8 | Jump if not above or equal |
| JBE disp8 | Jump if below or equal |
| JNA disp8 | Jump if not above |

Slide note: Before these instructions, a comparison instruction is to be used for comparing two operands (i.e. use CMP first).

Signed vs unsigned trick: Above/Below (A/B) are for unsigned data; Greater/Less (G/L) are for signed data.

Conditional branch instructions affecting INDIVIDUAL FLAGS:

| Mnemonic | Explanation |
| --- | --- |
| JC disp8 | Jump if CF = 1 |
| JNC disp8 | Jump if CF = 0 |
| JP disp8 | Jump if PF = 1 |
| JNP disp8 | Jump if PF = 0 |
| JO disp8 | Jump if OF = 1 |
| JNO disp8 | Jump if OF = 0 |
| JS disp8 | Jump if SF = 1 |
| JNS disp8 | Jump if SF = 0 |
| JZ disp8 | Jump if result is zero, i.e. ZF = 1 |
| JNZ disp8 | Jump if result is not zero, i.e. ZF = 0 |

## 7.3 Iteration control (loop) instructions

These instructions are used to execute the given instructions for a number of times.

| Instruction | Meaning |
| --- | --- |
| LOOP | Used to loop a group of instructions until the condition satisfies, i.e. CX = 0 |
| LOOPE / LOOPZ | Loop a group of instructions till it satisfies ZF = 1 & CX = 0 |
| LOOPNE / LOOPNZ | Loop a group of instructions till it satisfies ZF = 0 & CX = 0 |
| JCXZ | Used to jump to the provided address if CX = 0 |

Example (from the slides):

MOV CX, 06H label: ADD AL, BL LOOP label ; decrements CX (without affecting flags) and loops while CX ≠ 0

# 8. ASSEMBLER DIRECTIVES AND OPERATORS

## 8.1 What is an assembler directive?

•  Instructions to the assembler regarding the program being executed.

•  They control the generation of machine codes and organisation of the program; but no machine codes are generated for assembler directives.

•  Also called 'pseudo instructions'.

Used to:

•  specify the start and end of a program

•  attach value to variables

•  allocate storage locations to input/output data

•  define start and end of segments, procedures, macros, etc.

List of directives used in the slides: DB, DW, SEGMENT, ENDS, ASSUME, ORG, END, EVEN, EQU, PROC, FAR, NEAR, ENDP, SHORT, MACRO, ENDM

## 8.2 DB — Define Byte

•  Defines a byte type (8-bit) variable.

•  Reserves specific amount of memory locations to each variable.

•  Range: 00H – FFH for unsigned value; 00H – 7FH for positive value and 80H – FFH for negative value.

•  General form: variable DB value/values

•  Example:

LIST DB 7FH, 42H, 35H

→ Three consecutive memory locations are reserved for the variable LIST, and each data specified in the instruction is stored as the initial value in the reserved memory location.

## 8.3 DW — Define Word

•  Defines a word type (16-bit) variable.

•  Reserves two consecutive memory locations to each variable.

•  Range: 0000H – FFFFH for unsigned value; 0000H – 7FFFH for positive value and 8000H – FFFFH for negative value.

•  General form: variable DW value/values

•  Example:

ALIST DW 6512H, 0F251H, 0CDE2H

→ Six consecutive memory locations are reserved for the variable ALIST, each 16-bit data stored in two consecutive memory locations.

## 8.4 The remaining directives

| Directive | Meaning |
| --- | --- |
| SEGMENT | Used to indicate the beginning of a code/data/stack segment. General form: Segnam SEGMENT … Segnam ENDS (the user-defined name of the segment appears before SEGMENT and before ENDS). |
| ENDS | Used to indicate the end of a code/data/stack segment. |
| ASSUME | Informs the assembler the name of the program/data segment that should be used for a specific segment register. General form: ASSUME segreg : segnam, …, segreg : segnam — e.g. ASSUME CS: ACODE, DS: ADATA tells the compiler that the instructions of the program are stored in segment ACODE and data are stored in segment ADATA. |
| ORG | Origin — used to assign the starting address (Effective address) for a program/data segment. ORG 1000H informs the assembler that the statements following ORG 1000H should be stored in memory starting with effective address 1000H. |
| END | Used to terminate a program; statements after END will be ignored. |
| EVEN | Informs the assembler to store a program/data segment starting from an even address. |
| EQU | Equate — used to attach a value to a variable. LOOP EQU 10FEH → the value of variable LOOP is 10FEH. |
| PROC | Indicates the beginning of a procedure. procname PROC [NEAR/FAR] … RET … procname ENDP |
| ENDP | End of procedure. PROC and ENDP bracket a procedure. |
| FAR | Intersegment call (the procedure is in a different code segment). |
| NEAR | Intrasegment call (the procedure is in the same code segment). |
| SHORT | Reserves one memory location for an 8-bit signed displacement in jump instructions. JMP SHORT AHEAD → the directive reserves one memory location for the 8-bit displacement named AHEAD. |
| MACRO | Indicates the beginning of a macro. macroname MACRO [Arg1, Arg2 …] … ENDM |
| ENDM | End of a macro. |

Worked directive examples (from the slides):

_SDATA SEGMENT ORG 1200H A DB 4CH EVEN B DW 1052H _SDATA ENDS

→ In this data segment, the effective address of memory location assigned to A will be 1200H and that of B will be 1202H and 1203H.

ADD64 PROC NEAR ; the assembler will code the CALL and RET … ; instructions involved in this procedure RET ; as NEAR call and return ADD64 ENDP CONVERT PROC FAR ; the assembler will code CALL and RET … ; as FAR call and return RET CONVERT ENDP

## 8.5 Assembler operators: LENGTH, OFFSET, LEA

| Operator | Meaning |
| --- | --- |
| LENGTH | An operator which tells the assembler to determine the number of elements in some named data item, such as a string or an array. When the assembler reads MOV CX, LENGTH STRING1, it determines the number of elements in STRING1 and loads it into CX. If the string was declared as a byte string, LENGTH produces the number of bytes; if declared as a word string, LENGTH produces the number of words. Byte length of a label: used to refer to the length of a data array or a string — MOV CX, LENGTH ARRAY. |
| OFFSET | Offset of a label: when the assembler comes across the OFFSET operator along with a label, it first computes the 16-bit offset address of that particular label and replaces the string 'OFFSET LABEL' by the computed offset address. Example: MOV SI, OFFSET LIST |
| LEA | Load Effective Address — loads the address of a variable. Example: TEST DB 23H, 40H, 44H then LEA AX, TEST |

# 9. PROCEDURES, CALL/RET, MACROS AND MODULAR PROGRAMMING

## 9.1 Procedure

Definition: A procedure is a group of instructions that usually performs one task. It is a reusable section of a software program which is stored in memory once but can be used as often as necessary.

Two types:

•  NEAR procedure: a procedure is known as a NEAR procedure if it is written (defined) in the same code segment which is calling that procedure. Only the Instruction Pointer (IP) register content will be changed.

•  FAR procedure: a procedure is known as a FAR procedure if it is written (defined) in a different code segment than the calling segment. In this case both the IP and the Code Segment (CS) register contents will be changed.

Directives used for a procedure:

•  PROC directive: used to identify the start of a procedure; follows a name given to the procedure; after that the term FAR or NEAR specifies the type of the procedure.

•  ENDP directive: used along with the name of the procedure to indicate the end of a procedure to the assembler.

•  The PROC and ENDP directives are used to bracket a procedure.

## 9.2 CALL and RET instructions

CALL instruction: used to transfer execution to a procedure. It performs two operations:

1.  First it stores the address of the instruction after the CALL instruction on the stack.

2.  Second it changes the content of the IP register in case of a Near call and changes the content of the IP and CS registers in case of a FAR call.

Two types of calls: Near Call (Intrasegment call) and Far Call (Intersegment call).

Operation for Near Call: When the 8086 executes a near CALL instruction, it decrements the stack pointer by 2 and copies the IP register contents onto the stack. Then it copies the address of the first instruction of the called procedure (into IP).

Operation of FAR CALL: When the 8086 executes a far call, it:

1.  decrements the stack pointer by 2 and copies the contents of the CS register to the stack;

2.  decrements the stack pointer by 2 again and copies the content of the IP register to the stack;

3.  finally loads the CS register with the base address of the segment having the procedure and IP with the address of the first instruction in the procedure.

Stack order — FAR CALL pushes CS first, then IP (so RET/IRET can pop IP first). NEAR CALL pushes only IP.

## 9.3 Macro

•  MACRO Directive: indicates the beginning of a macro; ENDM: end of a macro.

•  General form:

macroname MACRO [Arg1, Arg2 ...] … program statements in the macro … ENDM

## 9.4 Procedure vs Macro (COMPARISON TABLE — high-yield)

| Procedure | Macro |
| --- | --- |
| Procedures are used for large group of instructions to be repeated | Macros are used for small group of instructions to be repeated |
| Object code is generated only once in memory | Object code is generated every time the macro is called |
| CALL & RET instructions are used to call procedure and return from procedure | Macro can be called just by writing its name |
| Length of the object file is less | Object file becomes lengthy |
| Directives PROC & ENDP are used for defining procedure | Directives MACRO and ENDM are used for defining MACRO |
| More time is required for its execution | Less time is required for its execution |
| Procedure can be defined as: Procedure_name PROC … Procedure_name ENDP | Macro can be defined as: MACRO-name MACRO [ARGUMENT, … ARGUMENT N] … ENDM |
| For example: Addition PROC near … Addition ENDP | For example: Display MACRO msg … ENDM |

## 9.5 Modular programming

Definition: Modular programming is a software design technique that emphasises separating the functionality of a program into independent, interchangeable modules, such that each contains everything necessary to execute only one aspect of the desired functionality.

•  A module interface expresses the elements that are provided and required by the module. The elements defined in the interface are detectable by other modules.

•  The implementation contains the working code that corresponds to the elements declared in the interface.

•  Modular programming is closely related to structured programming and object-oriented programming, all having the same goal of facilitating construction of large software programs and systems by decomposition into smaller pieces, and all originating around the 1960s.

•  While historical usage of these terms has been inconsistent, "modular programming" now refers to high-level decomposition of the code of an entire program into pieces; structured programming to the low-level code use of structured control flow; and object-oriented programming to the data use of objects (a kind of data structure).

# 10. EXAMPLE PROGRAMS

## 10.1 Procedure example program (with data + stack + code segments)

ASSUME CS:CODE, DS:DATA, SS:STACK_SEG DATA SEGMENT NUM1 DB 50H NUM2 DB 20H ADD_RES DB ? SUB_RES DB ? DATA ENDS STACK_SEG SEGMENT DW 40 DUP(0) ; stack of 40 words, all initialized to zero TOS LABEL WORD STACK_SEG ENDS CODE SEGMENT START: MOV AX, DATA ; initialize data segment MOV DS, AX MOV AX, STACK_SEG ; initialize stack segment MOV SS, AX MOV SP, OFFSET TOS ; initialize stack pointer to TOS CALL ADDITION CALL SUBTRACTION MOV AH, 4CH ; return to DOS INT 21H ADDITION PROC NEAR MOV AL, NUM1 MOV BL, NUM2 ADD AL, BL MOV ADD_RES, AL RET ADDITION ENDP SUBTRACTION PROC MOV AL, NUM1 MOV BL, NUM2 SUB AL, BL MOV SUB_RES, AL RET SUBTRACTION ENDP CODE ENDS END START

Points to write as comments in the exam:

•  MOV AX, DATA followed by MOV DS, AX — a segment register cannot be loaded with an immediate, so the segment base is first loaded into a general-purpose register (AX) and then moved to DS/SS.

•  MOV SP, OFFSET TOS initialises the stack pointer to the top of stack.

•  MOV AH, 4CH + INT 21H = DOS terminate program.

•  PROC NEAR … RET … ENDP bracket each procedure.

## 10.2 Macro example program

ASSUME CS:CODE, DS:DATA DATA SEGMENT NUM1 DW 1000H NUM2 DW 2000H RES DW ? DATA ENDS CODE SEGMENT ADDITION MACRO NO1, NO2, RESULT MOV AX, NO1 MOV BX, NO2 ADD AX, BX MOV RESULT, AX ENDM START: MOV AX, DATA ; initialize data segment MOV DS, AX ADDITION NUM1, NUM2, RES ; macro call — just write its name MOV AH, 4CH INT 21H CODE ENDS END START

## 10.3 Divide-by-zero demonstration (Emu8086 — Type-0 interrupt)

MOV AL, 05H MOV DL, 00H DIV DL ; → divide by zero → Type 0 interrupt HLT

## 10.4 Overflow demonstration (Type-4 interrupt)

A signed arithmetic overflow (the slides show an INT32 overflow exception in a high-level program) is handled through the Type-4 Overflow interrupt path — i.e. an arithmetic operation whose signed result does not fit the destination raises the overflow error condition.

# 11. MOST LIKELY EXAM QUESTIONS (INSTRUCTION SET)

| Question | Answer skeleton |
| --- | --- |
| Explain the instruction set of 8086 / classify the instructions of 8086. | The 6 groups: data transfer, arithmetic, logical, string manipulation, processor control, control transfer — with 2–3 example mnemonics each, then elaborate any one group. |
| Explain the data transfer instructions with examples (MOV, XCHG, PUSH, POP, IN, OUT). | The operation table of §2; mention MOV restrictions, PUSH/POP SP changes, IN/OUT via DX or addr8. |
| Explain MUL and DIV instructions with examples. | Byte: AL × operands → AX; Word: AX × operands → DX:AX. DIV: AX ÷ r8 → AL quotient, AH remainder; DX:AX ÷ r16 → AX quotient, DX remainder; mention Type-0 on divide overflow. |
| Differentiate AND and TEST. | Both perform the AND operation, but TEST does not change the first operand — used to check even/odd without altering the number. |
| Explain the shift and rotate instructions of 8086. | SHR, SHL/SAL, RCR, RCL, ROR, ROL with the CF/bit-flow table of §4; count can be 1 or CL only. |
| Explain the string instructions of 8086. | MOVS, CMPS, SCAS, LODS, STOS + REP/REPZ/REPNZ; source DS:SI, destination ES:DI; DF controls auto increment/decrement (1 for byte, 2 for word). |
| What does REP do? Difference between REPZ and REPNZ? | REP repeats while CX ≠ 0; REPZ/REPE repeats while CX ≠ 0 and ZF = 1; REPNZ/REPNE repeats while CX ≠ 0 and ZF = 0. |
| Explain the control transfer / branch instructions of 8086. | Unconditional (CALL, RET, JMP); signed conditional (JG/JGE/JL/JLE …); unsigned conditional (JA/JAE/JB/JBE …); flag-based (JC/JNC/JP/JO/JS/JZ …); note CMP is used first and control transfers modify IP within the same segment. |
| Explain the loop instructions with an example. | LOOP till CX = 0; LOOPE/LOOPZ till ZF = 1 & CX ≠ 0; LOOPNE/LOOPNZ till ZF = 0 & CX ≠ 0; JCXZ jumps if CX = 0 — with the MOV CX,06H / label: ADD AL,BL / LOOP label example. |
| What are assembler directives? Explain DB, DW, SEGMENT, ASSUME, ORG, EQU, PROC, ENDP, MACRO, ENDM. | Directives definition (pseudo instructions, no machine code generated) + the meaning table + the DB/DW range tables of §8. |
| Differentiate DB and DW. | DB = 8-bit variable, 1 location, 00H–FFH; DW = 16-bit variable, 2 consecutive locations, 0000H–FFFFH. |
| Explain LENGTH, OFFSET and LEA with examples. | LENGTH = number of elements of an array/string; OFFSET = 16-bit offset address of a label; LEA = load effective address of a variable. |
| Differentiate procedures and macros. | The full 7-row comparison table of §9.4. |
| Difference between NEAR and FAR procedure/call. | NEAR: same code segment, only IP pushed/changed. FAR: different segment, CS then IP pushed, both changed. |
| Write an ALP to add/subtract two numbers using a procedure. | The program of §10.1. |
| Write an ALP using a macro. | The program of §10.2. |
| What is modular programming? | Definition + interface vs implementation + relation to structured/OOP programming. |

# 12. LAST-MINUTE CRAM SHEET (INSTRUCTION SET)

The 6 groups: D-A-L-S-P-C = Data transfer, Arithmetic, Logical, String, Processor control, Control transfer.

Register rules to never forget:

| Situation | Result register |
| --- | --- |
| Byte MUL | AX ← AL × reg8/mem8 |
| Word MUL | DX:AX ← AX × reg16/mem16 |
| Word ÷ Byte | AL = quotient, AH = remainder |
| Dword ÷ Word | AX = quotient, DX = remainder |
| Shift/rotate count | `1` or `CL` only |
| Loop counter | CX |
| String source / destination | DS:SI / ES:DI |
| I/O indirect port address | DX only |

ADC/SBB: ADD/SUB with carry/borrow → used for multi-byte arithmetic. INC/DEC do not affect CF. CMP and TEST modify flags only — no operand change. Uses: AND = mask/clear bits · OR = set bits · XOR = toggle bits (XOR reg,reg clears) · TEST = check even/odd without modifying · SHL = ×2 · SHR(unsigned) = ÷2. DF in strings: DF = 0 → increment SI/DI; DF = 1 → decrement SI/DI (by 1 for byte, 2 for word). Conditional jumps: A/B family = unsigned; G/L family = signed; JZ/JNZ, JC/JNC, JP/JNP, JO/JNO, JS/JNS = individual flags. Use CMP first. Assembler directives generate NO machine code — they are pseudo instructions. PROC/ENDP bracket a procedure (CALL & RET, object code once) vs MACRO/ENDM bracket a macro (called by name, code expanded every time, faster but lengthier object file). FAR call pushes CS then IP; NEAR call pushes only IP.

Cross-check with Part 1 (Architecture) and Part 2 (Addressing Modes) — together the three files cover the complete Module 2 syllabus.