# MPMC — Module 2 — EXAM-READY NOTES

## 8086 Microprocessor: Architecture, Pin Diagram, Addressing Modes, Instruction Set, Assembler Directives & Interrupts

Scope: These notes cover Module 2 only (ECE3004 – Microprocessors and Microcontrollers). Every slide topic from the Module 2 deck is included: generations of microprocessors, 8086 overview, pins & signals (common/minimum/maximum mode), BIU–EU architecture, registers, flags, physical address calculation, all 12 addressing modes, all 6 instruction groups, string instructions, branch/loop instructions, assembler directives, procedures vs macros, modular programming and 8086 interrupts (IVT, interrupt processing, interrupt types).

This master file is also split into 3 topic-wise files (same content, easier to study/reprint):

•  Part 1 — Architecture → MPMC_Module2_Part1_Architecture.md

•  Part 2 — Addressing Modes → MPMC_Module2_Part2_Addressing_Modes.md

•  Part 3 — Instruction Set, Directives & Programming → MPMC_Module2_Part3_Instruction_Set.md

# 1. GENERATIONS OF MICROPROCESSORS

| Generation | Period | Technology | Word size / Pins | Key features | Example |
| --- | --- | --- | --- | --- | --- |
| First | 1971 – 1973 | PMOS (not TTL compatible) | 4-bit → 16 pins; 8- & 16-bit → 40 pins | Signals had to be multiplexed due to pin limitations | Intel 4004/4040 |
| Second | 1973 onward | NMOS (faster, higher density, TTL compatible) | 4 / 8 / 16-bit → 40 pins | Large memory & I/O addressing, greater subroutine nesting, better interrupt handling | Intel 8085 (8-bit) |
| Third | 1978 onward | HMOS (faster, higher packing density) | 16-bit → 40 / 48 / 64 pins | Easier to program, dynamically relocatable programs, multiply/divide hardware, powerful interrupts, flexible I/O addressing | Intel 8086 (16-bit) |
| Fourth | 1980s | HCMOS (low-power HMOS) | 32-bit | Physical memory 2²⁴ = 16 MB, virtual memory 2⁴⁰ = 1 TB, floating-point hardware, many addressing modes | Intel 80386 |
| Fifth | — | — | 32/64-bit | Advanced pipelining, superscalar | Pentium |

Memory trick: P-N-H-H-P = PMOS → NMOS → HMOS → HCMOS → Pentium.

# 2. GENERAL MICROPROCESSOR FUNCTIONAL BLOCKS

The general block diagram of any microprocessor has:

| Block | Function |
| --- | --- |
| ALU (Computational Unit) | Performs arithmetic and logic operations |
| Flag Register | Stores various conditions of the result as status bits (flags) |
| Register array / internal memory | Internal storage of data |
| PC / IP | Generates the address of the next instruction to be fetched from memory, sent via the address bus |
| Instruction decoding unit | Decodes instructions; sends information to the timing and control unit |
| Timing and control unit | Generates control signals for internal and external operations of the µ P |

Buses: Control bus, Address bus, Data bus connect these blocks to memory and I/O.

# 3. 8086 MICROPROCESSOR — OVERVIEW & SALIENT FEATURES

•  First 16-bit processor released by INTEL in 1978.

•  Originally HMOS; now manufactured using HMOS III technique.

•  ≈29,000 transistors, 40-pin DIP, +5 V supply.

•  No internal clock generator → needs an external asymmetric clock with 33 % duty cycle (from the 8284A clock generator).

•  20-bit address bus → can address 2²⁰ = 1 MB (1 megabyte) of memory.

•  16-bit data bus → word size is 16 bits.

•  Works in two modes: Minimum mode (single processor) and Maximum mode (multiprocessor/coprocessor).

Quick comparison (Module-2 favourites):

| Feature | 8085 | 8086 |
| --- | --- | --- |
| Word length | 8-bit | 16-bit |
| Address bus | 16-bit | 20-bit |
| Memory | 64 KB | 1 MB |
| Clock | 3 MHz | 5 / 8 / 10 MHz (varies) |
| Multiplexing | Lower address/data | Address/data + status multiplexed |
| Pipelining | No | Yes (6-byte queue) |

# 4. 8086 PIN DIAGRAM AND SIGNALS (40-pin DIP)

Pin layout (memorise the groups, not all 40 numbers):

| Pins | Name |
| --- | --- |
| 1, 20 | GND (ground) |
| 2–16, 39 | AD0–AD15 (address/data bus, bidirectional, multiplexed) |
| 35–38 | A16/S3, A17/S4, A18/S5, A19/S6 |
| 17 | NMI |
| 18 | INTR |
| 19 | CLK |
| 21 | RESET |
| 22 | READY |
| 23 | TEST |
| 24 | QS1, 25 = QS0 (max mode) / 25 = ALE (min mode) |
| 26 | DEN (min) / S0 (max) |
| 27 | DT/R (min) / S1 (max) |
| 28 | M/IO (min) / S2 (max) |
| 29 | WR (min) / LOCK (max) |
| 30 | HLDA (min) / RQ/GT1 (max) |
| 31 | HOLD (min) / RQ/GT0 (max) |
| 32 | RD |
| 33 | MN/MX (minimum/maximum mode select) |
| 34 | BHE/S7 |
| 40 | VCC |

Exam tip: The same pin carries different names in minimum and maximum modes — this is the single most repeated pin question.

## 4.1 Common Signals (both modes)

| Signal | Direction | Description |
| --- | --- | --- |
| AD0–AD15 | Bidirectional, multiplexed | Low-order address bus multiplexed with data. Written A0–A15 when carrying addresses, D0–D15 when carrying data. |
| A16/S3 – A19/S6 | Output, multiplexed | High-order address bus multiplexed with status signals. |
| BHE/S7 (active low) | Output | Bus High Enable: enables data onto the most significant half of the data bus (D8–D15). 8-bit devices connected to the upper half of the data bus use BHE. Multiplexed with status S7. |
| MN/MX | Input | 1 → Minimum mode; 0 → Maximum mode (indicates what mode the processor is to operate in). |
| RD (active low) | Output | Read operation; active low. |
| READY | Input, active HIGH | Acknowledgement from a slow device/memory that it has completed the data transfer. Synchronised by the 8284A clock generator before being given to the 8086. |
| RESET | Input, active HIGH | Immediately terminates the present activity; must be active HIGH for at least 4 clock cycles. |
| CLK | Input | Basic timing for processor operation and bus control activity; an asymmetric square wave with 33 % duty cycle. |
| INTR | Input, active HIGH, level triggered | Interrupt Request, sampled during the last clock cycles of each instruction to determine availability of the request. If pending → processor enters the interrupt acknowledge cycle. Internally synchronised. |
| NMI | Input | Non-maskable interrupt (positive edge triggered). |
| TEST | Input | Used with WAIT; if TEST = 0 the processor continues, else waits. |

Note: The 8086 does not have an on-chip clock generation circuit → the 8284 clock generator is connected to the CLK pin.

## 4.2 Minimum-Mode Signals (MN/MX = 1)

| Signal | Description |
| --- | --- |
| ALE (Address Latch Enable) | Demultiplexes the address/data lines using external latches (8282). A positive pulse is generated each time the processor begins any operation; indicates a valid address on the address/data lines. Also differentiates memory access from I/O access (high for memory reference, low for IN/OUT). |
| DEN (Data Enable) | Output enable for the 8286 transceiver. Also called Data Enable; enables the transceiver that separates data from the address/data bus. |
| DT/R (Data Transmit/Receive) | Controls the direction of data flow through the data transceivers. |
| M/IO | Used to differentiate memory access and I/O access: high for memory reference, low for IN and OUT instructions. |
| WR (active low) | Write control signal; asserted low whenever the processor writes data to memory or I/O port. |
| INTA (active low) | Interrupt Acknowledge: becomes low on this line when the interrupt request is accepted by the processor. |
| HOLD | Input from a bus master requesting control of the bus (usually by the DMA controller). |
| HLDA (Hold Acknowledge) | Acknowledge by the processor to the bus master via HOLD; asserted high when the processor accepts HOLD. |

Demultiplexing hardware (very common exam figure): 8282 latch (strobed by ALE) separates A0–A19 (and gives BHE), while the 8286 transceiver (controlled by DT/R and DEN) handles D0–D15. That's how the multiplexed AD bus is converted into separate address and data buses.

## 4.3 Maximum-Mode Signals (MN/MX = 0)

| Signal | Description |
| --- | --- |
| S0, S1, S2 (active low) | Status signals used by the 8288 bus controller to generate bus timing and control signals. |
| QS0, QS1 | Queue Status: tell the status of the instruction queue; external devices can track the internal queue status. |
| LOCK (active low) | Locks the bus during the next instruction; prevents other bus masters from taking the bus. |
| RQ/GT0, RQ/GT1 (Request/Grant) | Used to request and grant bus access by other processors (e.g., 8087 coprocessor) — bidirectional. |

S2 S1 S0 → Machine Cycle (MUST MEMORISE):

| S2 | S1 | S0 | Machine Cycle |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Interrupt acknowledge |
| 0 | 0 | 1 | Read I/O port |
| 0 | 1 | 0 | Write I/O port |
| 0 | 1 | 1 | Halt |
| 1 | 0 | 0 | Code access (instruction fetch) |
| 1 | 0 | 1 | Read memory |
| 1 | 1 | 0 | Write memory |
| 1 | 1 | 1 | Passive / Inactive |

QS1 QS0 → Queue operation:

| QS1 | QS0 | Queue operation |
| --- | --- | --- |
| 0 | 0 | No operation |
| 0 | 1 | First byte of an opcode from queue |
| 1 | 0 | Empty the queue |
| 1 | 1 | Subsequent byte from queue |

## 4.4 Minimum vs Maximum Mode

| Point | Minimum Mode | Maximum Mode |
| --- | --- | --- |
| MN/MX pin | High (1) | Low (0) |
| Coprocessor use | Not associated with any coprocessor | Works in multiprocessor / coprocessor configuration |
| Bus controller | Not needed (8086 gives all signals itself) | 8288 bus controller required |
| Signals generated | 8086 generates ALE, DEN, DT/R, M/IO, WR, INTA, HOLD, HLDA | 8086 gives S0–S2, QS0–QS1, LOCK, RQ/GT0–RQ/GT1 |
| Typical use | Single-processor small systems | Multiprocessor systems (e.g., 8086 + 8087 + 8089) |

# 5. INSIDE THE 8086 — THE ARCHITECTURE (BIU + EU)

## 5.1 Pipelining

•  Two ways to make a CPU faster:

1.  Increase working frequency — technology dependent.

2.  Change the internal architecture of the CPU ← what Intel did.

•  Pipelining = allowing the CPU to fetch and execute at the same time.

•  Intel implemented it by splitting the 8086 into two independent sections that work simultaneously:

•  Execution Unit (EU) — executes instructions previously fetched.

•  Bus Interface Unit (BIU) — accesses memory and peripherals.

•  Benefit: instruction fetch is overlapped with execution → speed-up.

## 5.2 Bus Interface Unit (BIU)

Functions: fetches instructions, reads data from memory and I/O ports, writes data to memory and I/O ports.

Contains:

1.  Dedicated adder to generate the 20-bit physical address.

2.  Four 16-bit segment registers — CS, DS, SS, ES.

3.  Instruction Pointer (IP).

4.  6-byte instruction queue (FIFO).

Segment registers details:

| Register | Points to | Offset used | Notes |
| --- | --- | --- | --- |
| CS (Code Segment) | Base/start of current code segment | IP | 20-bit physical address = CS shifted 4 bits left + IP. All instructions of a program are relative to CS×16 plus the offset provided by IP. |
| DS (Data Segment) | Current data segment | SI / DI / 16-bit displacement | Operands for most instructions are fetched from this segment. |
| SS (Stack Segment) | Current stack | SP (PUSH/POP), BP (based addressing) | Physical stack address = SS×16 + SP (or + BP). |
| ES (Extra Segment) | Extra segment; data in excess of 64 K pointed by DS | DI (string destination) | String instructions use ES:DI for the destination address. |

Memory segmentation facts:

•  8086's 1 MB memory is divided into segments of up to 64 KB each.

•  Programs obtain access to code and data by changing the segment register contents to point to the desired segment.

•  At any particular time, the 8086 can directly address 4 segments (256 KB within the 1 MB).

Instruction Queue:

•  A group of First-In-First-Out (FIFO) registers holding up to 6 bytes of instruction code pre-fetched from memory ahead of time.

•  Done to speed up execution by overlapping instruction fetch with execution — this mechanism is pipelining.

Instruction Pointer (IP):

•  16-bit; always points to the next instruction to be executed within the currently executing code segment.

•  Contains the 16-bit offset address of the next instruction code within the 64 KB code segment.

•  Automatically incremented as execution proceeds.

## 5.3 Execution Unit (EU)

Functions: decodes and executes instructions. A decoder in the EU control system translates instructions.

Contains:

•  16-bit ALU for arithmetic and logic operations.

•  Four general-purpose registers: AX, BX, CX, DX (each 16-bit; splittable into two 8-bit halves).

•  Pointer registers: SP, BP.

•  Index registers: SI, DI.

•  Flag register.

8-bit splitting rule: AX→(AH, AL), BX→(BH, BL), CX→(CH, CL), DX→(DH, DL). The low-order byte goes to AL/BL/CL/DL and the high-order byte to AH/BH/CH/DH.

Exam one-liner: BIU fetches, EU executes — and they work simultaneously → pipelining.

# 6. REGISTERS OF 8086 (WITH SPECIAL FUNCTIONS)

| Sl. | Type | Width | Registers |
| --- | --- | --- | --- |
| 1 | General purpose | 16-bit / 8-bit | AX, BX, CX, DX / AL, AH, BL, BH, CL, CH, DL, DH |
| 2 | Pointer | 16-bit | SP, BP |
| 3 | Index | 16-bit | SI, DI |
| 4 | Instruction Pointer | 16-bit | IP |
| 5 | Segment | 16-bit | CS, DS, SS, ES |
| 6 | Flag (PSW) | 16-bit | Flag register |

| Register | Name | Special Function |
| --- | --- | --- |
| AX | 16-bit Accumulator | Stores 16-bit results of arithmetic & logic operations |
| AL | 8-bit Accumulator | Stores 8-bit results of arithmetic & logic operations |
| BX | Base register | Holds base value in base addressing mode to access memory data. Only general-purpose register whose contents can be used for addressing memory; memory references using BX default to DS. |
| CX | Count register | Holds count value for SHIFT, ROTATE and LOOP instructions |
| DX | Data register | Holds data for multiplication and division operations; also holds the 16-bit port address for indirect I/O |
| SP | Stack Pointer | Holds the offset address of the top of stack memory |
| BP | Base Pointer | Holds base value in based addressing using SS to access data from stack memory |
| SI | Source Index | Holds index value of source operand (data) for string instructions |
| DI | Destination Index | Holds index value of destination operand (data) for string operations |

Extra notes from slides:

•  AX/AL are used by I/O instructions (IN/OUT) for 16/8-bit data, and by MUL/DIV instructions.

•  CX example: LOOP START automatically decrements CX by 1 without affecting flags and checks if CX = 0. If zero, the 8086 executes the next instruction; otherwise it branches to the label START.

•  SI & DI are used in indexed addressing; string instructions use SI with DS and DI with ES to distinguish source and destination addresses.

## 6.1 Flag Register (PSW) — bit by bit

| Bit | Flag | Name | Set (=1) when… |
| --- | --- | --- | --- |
| 0 | CF | Carry Flag | There is a carry out of MSB in addition or a borrow in subtraction |
| 2 | PF | Parity Flag | Lower byte of result contains an even number of 1's (else 0) |
| 4 | AF | Auxiliary Carry Flag | There is a carry from the lowest nibble (bit 3) during addition, or borrow from bit 3 during subtraction |
| 6 | ZF | Zero Flag | Result of computation/comparison is zero |
| 7 | SF | Sign Flag | Result of computation is negative |
| 8 | TF | Trap Flag | Set → processor enters single-step execution mode (generates internal interrupts after each instruction) |
| 9 | IF | Interrupt Flag | Controls recognition of external maskable interrupts; clearing IF disables these interrupts |
| 10 | DF | Direction Flag | Used by string instructions: DF = 0 → auto-incrementing (lowest → highest address); DF = 1 → auto-decrementing (highest → lowest address) |
| 11 | OF | Overflow Flag | Result of a signed operation is too large to fit in the destination (more than 7 bits for 8-bit, more than 15 bits for 16-bit signed operations) |

Bit positions (memorise the layout):

15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0 OF DF IF TF SF ZF AF PF CF

Note: Bits 1, 3, 5, 12, 13, 14, 15 are undefined/unused in the 8086.

# 7. PHYSICAL ADDRESS GENERATION (MEMORY ACCESS)

Why needed? 8086 has 20 address lines (→ 2²⁰ = 1 MB) but the largest register is only 16 bits.

Definitions:

•  Physical Address: the actual address of a byte in memory, i.e. the value which goes out onto the address bus (20 bits).

•  Memory addresses are represented as Segment : Offset — e.g. 89AB:F012.

Formula:

Physical Address = (Segment register × 16₁₀) + Offset

= shift the segment register one hex digit left (append 0) and add the offset.

Worked example (from slides) — 89AB:F012

Segment 89AB → 89AB0 (paragraph to byte: 89AB × 10H = 89AB0) Offset F012 → F012 (offset is already in byte units) ----- Physical address 98AC2 H

(16 bytes of contiguous memory = one "paragraph".)

Typical segment : offset pairs used:

| Access type | Segment | Offset |
| --- | --- | --- |
| Instruction fetch | CS | IP |
| Data (stack ops) | SS | SP |
| Data (based) | SS | BP |
| Data (default) | DS | BX / SI / DI / disp |
| String source | DS | SI |
| String destination | ES | DI |

Addressing-modes notation used in the slides (write these in the exam):

•  EA = Effective Address (offset part), BA = Base Address (segment × 16₁₀), MA = Memory Address = BA + EA.

# 8. ADDRESSING MODES OF 8086

Definition: Every instruction of a program operates on data. The different ways in which a source operand is denoted in an instruction are known as addressing modes.

The 12 modes, grouped as in the slides:

| Group | Modes |
| --- | --- |
| Group I — register & immediate data | 1. Register, 2. Immediate |
| Group II — memory data | 3. Direct, 4. Register Indirect, 5. Based, 6. Indexed, 7. Based-Index, 8. String |
| Group III — I/O ports | 9. Direct I/O port, 10. Indirect I/O port |
| Group IV — relative | 11. Relative |
| Group V — implied | 12. Implied |

## 8.1 Register Addressing

The instruction specifies the name of the register which holds the data.

MOV CL, DH (CL) ← (DH) ADD AX, BX (AX) ← (AX) + (BX)

Content of source is NOT cleared after execution.

## 8.2 Immediate Addressing

An 8-bit or 16-bit data is specified as part of the instruction.

MOV DL, 08H (DL) ← 08H SUB AX, 0A9FH (AX) ← (AX) – 0A9FH

## 8.3 Direct Addressing (Group II)

The effective address of the memory location at which the data operand is stored is given directly in the instruction (16-bit displacement written directly).

MOV [2000H], 02H MOV [0234H], 0234H MOV BX, [1354H] ; copy contents of memory location 1354H into BX MOV [1234H], BX MOV BL, [1354H] MOV [0400H], CL

The square brackets around 1354H denote the contents of the memory location.

## 8.4 Register Indirect Addressing

The name of the register which holds the EA is specified. Registers used: BX, BP, DI, SI. DS is used for the base address by default.

MOV CX, [BX]

EA = (BX) BA = (DS) × 16₁₀ MA = BA + EA (CX) ← (MA) or (CL) ← (MA), (CH) ← (MA+1)

## 8.5 Based Addressing

BX or BP holds the base value, plus a signed 8-bit or unsigned 16-bit displacement. An 8-bit displacement is sign-extended to 16-bit before adding.

•  BX with DS → physical address from BX and DS.

•  BP with SS → physical address from BP and SS.

MOV AX, [BX + 08H] MOV [BX + 1234H], AX MOV [BP + 21H], 9878H

0008H ← 08H (sign extended) ; EA = (BX) + 0008H ; BA = (DS) × 16₁₀ ; MA = BA + EA (AX) ← (MA) [ (AL) ← (MA), (AH) ← (MA+1) ]

## 8.6 Indexed Addressing

SI or DI holds an index value, plus a signed 8-bit or unsigned 16-bit displacement (added to get EA).

MOV CX, [SI + 0A2H]

FFA2H ← A2H (sign extended) ; EA = (SI) + FFA2H ; BA = (DS) × 16₁₀ ; MA = BA + EA (CX) ← (MA)

## 8.7 Based-Index Addressing

EA = base register (BX or BP) + index register (SI or DI) + displacement.

MOV DX, [BX + SI + 0AH]

000AH ← 0AH (sign extended) ; EA = (BX) + (SI) + 000AH ; BA = (DS) × 16₁₀ ; MA = BA + EA (DX) ← (MA)

Legal register combinations for memory operands (learn the table):

Allowed combinations

[BX + SI], [BX + DI], [BP + SI], [BP + DI]

[SI], [DI], [BX], [BP]

[BX + SI + d8], [BX + DI + d8], [BP + SI + d8], [BP + DI + d8]

[SI + d8], [DI + d8], [BP + d8], [BX + d8]

[BX + SI + d16], [BX + DI + d16], [BP + SI + d16], [BP + DI + d16]

[SI + d16], [DI + d16], [BP + d16], [BX + d16]

d16 (variable offset only — direct addressing)

Only BX, BP, SI, DI can go inside [ ]. BX/BP = base (BP→SS), SI/DI = index.

## 8.8 String Addressing

Employed in string operations.

•  EA of source data → SI register; EA of destination → DI register.

•  Segment for source = DS; segment for destination = ES.

MOVS BYTE Source: EA = (SI) BA = (DS) × 16₁₀ MA = BA + EA Destination: EAE = (DI) BAE = (ES) × 16₁₀ MAE = BAE + EAE (MAE) ← (MA) If DF = 1 → (SI) ← (SI) – 1 ; (DI) ← (DI) – 1 If DF = 0 → (SI) ← (SI) + 1 ; (DI) ← (DI) + 1

## 8.9 Direct I/O Port Addressing (Group III)

An 8-bit port address is directly specified in the instruction.

IN AL, [09H] PORTaddr = 09H ; (AL) ← (PORT)

## 8.10 Indirect I/O Port Addressing

The instruction specifies the register which holds the port address — in 8086 the 16-bit port address is stored in DX.

OUT [DX], AX PORTaddr = (DX) ; (PORT) ← (AX)

## 8.11 Relative Addressing (Group IV)

The EA of a program instruction is specified relative to the Instruction Pointer (IP) by an 8-bit signed displacement.

JZ 0AH 000AH ← 0AH (sign extend) If ZF = 1 → EA = (IP) + 000AH ; BA = (CS) × 16₁₀ ; MA = BA + EA → jump If ZF = 0 → next instruction is executed

## 8.12 Implied Addressing (Group V)

Instructions using this mode have no operands — the instruction itself specifies the data.

CLC ; clears the carry flag to zero (CF ← 0)

# 9. INSTRUCTION SET OF 8086

8086 supports 6 types of instructions:

1.  Data Transfer Instructions

2.  Arithmetic Instructions

3.  Logical Instructions

4.  String Manipulation Instructions

5.  Processor Control Instructions

6.  Control Transfer Instructions

Notation: ( ) denotes contents of; ← denotes transfer; reg2 = destination, reg1 = source (as used in the slides).

## 9.1 Data Transfer Instructions

Instructions used to transfer data/address into registers, memory locations and I/O ports. Generally two operands (source & destination) of the same size. Size must be a byte or a word; an 8-bit data can move only to an 8-bit register/memory and 16-bit data to a 16-bit register/memory.

Mnemonics: MOV, XCHG, PUSH, POP, IN, OUT

| Instruction | Operation |
| --- | --- |
| MOV reg2, reg1 | (reg2) ← (reg1) |
| MOV mem, reg1 | (mem) ← (reg1) |
| MOV reg2, mem | (reg2) ← (mem) |
| MOV reg, data | (reg) ← data |
| MOV mem, data | (mem) ← data |
| XCHG reg2, reg1 | (reg2) ↔ (reg1) |
| XCHG mem, reg1 | (mem) ↔ (reg1) |
| PUSH reg16 / mem | (SP) ← (SP) – 2 ; MA_S = (SS)×16₁₀ + SP ; (MA_S ; MA_S+1) ← (reg16)/(mem) |
| POP reg16 / mem | MA_S = (SS)×16₁₀ + SP ; (reg16)/(mem) ← (MA_S ; MA_S+1) ; (SP) ← (SP) + 2 |
| IN AL, [DX] | PORTaddr = (DX) ; (AL) ← (PORT) |
| IN AX, [DX] | PORTaddr = (DX) ; (AX) ← (PORT) |
| IN AL, addr8 | (AL) ← (addr8) |
| IN AX, addr8 | (AX) ← (addr8) |
| OUT [DX], AL | PORTaddr = (DX) ; (PORT) ← (AL) |
| OUT [DX], AX | PORTaddr = (DX) ; (PORT) ← (AX) |
| OUT addr8, AL | (addr8) ← (AL) |
| OUT addr8, AX | (addr8) ← (AX) |

Remember: MOV cannot transfer memory → memory or segment register → immediate.

## 9.2 Arithmetic Instructions

Mnemonics: ADD, ADC, SUB, SBB, INC, DEC, MUL, DIV, IMUL, IDIV, CMP

| Instruction | Operation |
| --- | --- |
| ADD reg2, reg1 | (reg2) ← (reg1) + (reg2) |
| ADD reg2, mem | (reg2) ← (reg2) + (mem) |
| ADD mem, reg1 | (mem) ← (mem) + (reg1) |
| ADD reg, data | (reg) ← (reg) + data |
| ADD AL, data8 | (AL) ← (AL) + data8 |
| ADD AX, data16 | (AX) ← (AX) + data16 |
| ADC … | Same as ADD but adds CF: (reg2) ← (reg1) + (reg2) + CF |
| SUB reg2, reg1 | (reg2) ← (reg1) – (reg2) |
| SUB reg, data | (reg) ← (reg) – data |
| SBB … | Same as SUB but subtracts CF: (reg) ← (reg) – data – CF |
| INC reg8/reg16/mem | (reg8/16) ← (…)+1 ; (mem) ← (mem)+1 |
| DEC reg8/reg16/mem | (reg8/16) ← (…)–1 ; (mem) ← (mem)–1 |
| MUL reg (byte) | (AX) ← (AL) × (reg8) (result 16-bit) |
| MUL reg (word) | (DX)(AX) ← (AX) × (reg16) (result 32-bit) |
| MUL mem | Same pattern with (mem8)/(mem16) |
| IMUL reg/mem | Signed multiply — same register conventions as MUL |
| DIV reg (16÷8) | (AL) ← quotient, (AH) ← remainder |
| DIV reg (32÷16) | (AX) ← quotient, (DX) ← remainder |
| IDIV reg/mem | Signed divide — same conventions as DIV |
| CMP reg2, reg1 | Modify flags ← (reg2) – (reg1) |

CMP flag results (memorise!):

| Condition | CF | ZF | SF |
| --- | --- | --- | --- |
| (reg2) > (reg1) | 0 | 0 | 0 |
| (reg2) < (reg1) | 1 | 0 | 1 |
| (reg2) = (reg1) | 0 | 1 | 0 |

(Same table applies for `CMP reg2, mem`, `CMP mem, reg1`, `CMP reg, data`, `CMP mem, data`, `CMP AL, data8`, `CMP AX, data16`.)

Critical exam point — MUL/DIV operand rules:

•  Byte MUL: AL × reg8/mem8 → AX (16-bit product)

•  Word MUL: AX × reg16/mem16 → DX:AX (32-bit product)

•  Word ÷ Byte: AX ÷ reg8 → AL = quotient, AH = remainder

•  Dword ÷ Word: DX:AX ÷ reg16 → AX = quotient, DX = remainder

•  If the quotient cannot fit (divide overflow / divide by zero) → INT 0 (Type-0 interrupt) is generated.

## 9.3 Logical Instructions

Mnemonics: AND, OR, XOR, NOT, TEST, SHR, SHL/SAL, RCR, RCL, ROR, ROL

| Instruction | Operation |
| --- | --- |
| AND reg2, reg1 | (reg2) ← (reg2) & (reg1) |
| AND reg2, mem | (reg2) ← (reg2) & (mem) |
| AND mem, reg1 | (mem) ← (mem) & (reg1) |
| AND reg, data | (reg) ← (reg) & data |
| AND AL, data8 / AND AX, data16 | (AL) ← (AL) & data8 / (AX) ← (AX) & data16 |
| OR reg2, reg1 | (reg2) ← (reg2) ¦ (reg1) |
| OR reg, data | (reg) ← (reg) ¦ data |
| OR mem, data | (mem) ← (mem) ¦ data |
| OR AL, data8 / OR AX, data16 | (AL) ← (AL) ¦ data8 / (AX) ← (AX) ¦ data16 |
| XOR reg2, reg1 | (reg2) ← (reg2) ^ (reg1) |
| XOR reg2, mem | (reg2) ← (reg2) ^ (mem) |
| XOR mem, reg1 | (mem) ← (mem) ^ (reg1) |
| XOR reg, data | (reg) ← (reg) ^ data |
| XOR AL, data8 / XOR AX, data16 | (AL) ← (AL) ^ data8 / (AX) ← (AX) ^ data16 |
| TEST reg2, reg1 | Modify flags ← (reg2) & (reg1) — operands unchanged |
| TEST mem, reg1 | Modify flags ← (mem) & (reg1) |
| TEST reg, data | Modify flags ← (reg) & data |
| TEST AL, data8 / TEST AX, data16 | Modify flags ← (AL) & data8 / (AX) & data16 |

TEST vs AND: TEST works the same as AND but does not change the first operand. To check whether a number in a register is even or odd, TEST can be used without changing the original number.

Shift instructions:

| Instruction | Operation |
| --- | --- |
| SHR reg/mem, 1 or SHR reg/mem, CL | CF ← B_LSD ; B_n ← B_n+1 ; B_MSD ← 0 (logical shift right; MSB filled with 0) |
| SHL reg/mem, 1 (or SAL) | CF ← B_MSD ; B_n+1 ← B_n ; B_LSD ← 0 (logical shift left; LSB filled with 0) |

(Diagrams in the slides show the bit movement for 8-bit and 16-bit operands with CF at the end.)

Rotate instructions:

| Instruction | Operation |
| --- | --- |
| RCR reg/mem, 1 / , CL | B_n ← B_n–1 ; B_MSD ← CF ; CF ← B_LSD (rotate through carry right) |
| RCL | rotate through carry left (complement of RCR) |
| ROL reg/mem, 1 / , CL | B_n–1 ← B_n ; CF ← B_MSD ; B_LSD ← B_MSD (rotate left, MSB copied to LSB and to CF) |
| ROR | rotate right (complement of ROL) |

Count rule: shift/rotate can use either `1` (immediate) or `CL` (variable count) — never any other register.

## 9.4 String Manipulation Instructions

•  String: a sequence of bytes or words.

•  8086 includes instructions for string movement, comparison, scan, load and store.

•  REP prefix repeats execution of string instructions.

•  String mnemonics end with S / SB / SW — S = String, SB = String Byte, SW = String Word.

•  Source offset/EA → SI register; destination offset/EA → DI register.

•  Depending on DF, SI and DI are automatically updated:

•  DF = 0 → incremented by 1 for byte, 2 for word.

•  DF = 1 → decremented by 1 for byte, 2 for word.

Mnemonics: REP, MOVS, CMPS, SCAS, LODS, STOS

| Prefix | Meaning |
| --- | --- |
| REP | Repeat while CX ≠ 0, and (CX) ← (CX) – 1 |
| REPZ / REPE | Repeat CMPS or SCAS while CX ≠ 0 and ZF = 1; (CX) ← (CX) – 1 (result should be zero for condition true) |
| REPNZ / REPNE | Repeat CMPS or SCAS while CX ≠ 0 and ZF = 0; (CX) ← (CX) – 1 (result should not be zero for condition true) |

Example: `repz movsb`

| Instruction | Operation |
| --- | --- |
| MOVS (MOVSB, MOVSW) | MA = (DS)×16₁₀ + (SI) ; MAE = (ES)×16₁₀ + (DI) ; (MAE) ← (MA). For MOVSW: (MAE;MAE+1) ← (MA;MA+1). DF=0 → SI,DI +1 (or +2 for word); DF=1 → SI,DI –1 (or –2). |
| CMPS (CMPSB, CMPSW) | Compare two string bytes/words. Modify flags ← (MA) – (MAE). If (MA) > (MAE): CF=0, ZF=0, SF=0; if <: CF=1, ZF=0, SF=1; if =: CF=0, ZF=1, SF=0. SI/DI updated ±1 (byte) or ±2 (word) as per DF. |
| SCAS (SCASB, SCASW) | Scan (compare) a string byte/word with the accumulator. MAE = (ES)×16₁₀ + (DI); Modify flags ← (AL) – (MAE) (or (AX) – (MAE;MAE+1) for word). DI updated ±1 / ±2 as per DF. |
| LODS (LODSB, LODSW) | Load string byte into AL / string word into AX. MA = (DS)×16₁₀ + (SI); (AL) ← (MA) [or (AX) ← (MA;MA+1)]; SI updated ±1 / ±2 as per DF. |
| STOS (STOSB, STOSW) | Store byte from AL / word from AX into string. MAE = (ES)×16₁₀ + (DI); (MAE) ← (AL) [or (MAE;MAE+1) ← (AX)]; DI updated ±1 / ±2 as per DF. |

## 9.5 Processor Control Instructions

| Mnemonic | Explanation |
| --- | --- |
| STC | Set carry: CF ← 1 |
| CLC | Clear carry: CF ← 0 |
| CMC | Complement carry: CF ← CF′ |
| STD | Set direction flag: DF ← 1 |
| CLD | Clear direction flag: DF ← 0 |
| STI | Set interrupt enable flag: IF ← 1 |
| CLI | Clear interrupt enable flag: IF ← 0 |
| NOP | No operation |
| HLT | Halt after interrupt is set |
| WAIT | Wait for TEST pin active |
| ESC opcode mem/reg | Pass instruction to a coprocessor which shares the address and data bus with the 8086 |
| LOCK | Lock bus during next instruction |

Note: STI/CLI control maskable interrupts only — NMI can never be masked.

## 9.6 Control Transfer Instructions

•  Transfer control to a specific destination/target instruction.

•  They do NOT affect flags.

Unconditional transfers:

| Mnemonic | Explanation |
| --- | --- |
| CALL reg/mem/disp16 | Call subroutine |
| RET | Return from subroutine |
| JMP reg/mem/disp8/disp16 | Unconditional jump |

Conditional branch instructions — SIGNED (after CMP, for signed data):

| Name | Alternate name |
| --- | --- |
| JE disp8 | Jump if equal |
| JZ disp8 | Jump if result is 0 |
| JNE disp8 | Jump if not equal |
| JNZ disp8 | Jump if not zero |
| JG disp8 | Jump if greater |
| JNLE disp8 | Jump if not less or equal |
| JGE disp8 | Jump if greater than or equal |
| JNL disp8 | Jump if not less |
| JL disp8 | Jump if less than |
| JNGE disp8 | Jump if not greater than or equal |
| JLE disp8 | Jump if less than or equal |
| JNG disp8 | Jump if not greater |

Conditional branch instructions — UNSIGNED (after CMP, for unsigned data):

| Name | Alternate name |
| --- | --- |
| JE disp8 | Jump if equal |
| JZ disp8 | Jump if result is 0 |
| JNE disp8 | Jump if not equal |
| JNZ disp8 | Jump if not zero |
| JA disp8 | Jump if above |
| JNBE disp8 | Jump if not below or equal |
| JAE disp8 | Jump if above or equal |
| JNB disp8 | Jump if not below |
| JB disp8 | Jump if below |
| JNAE disp8 | Jump if not above or equal |
| JBE disp8 | Jump if below or equal |
| JNA disp8 | Jump if not above |

Note (slide): Before these instructions, a comparison instruction must be used for comparing two operands.

Conditional branch instructions affecting individual flags:

| Mnemonic | Explanation |
| --- | --- |
| JC disp8 | Jump if CF = 1 |
| JNC disp8 | Jump if CF = 0 |
| JP disp8 | Jump if PF = 1 |
| JNP disp8 | Jump if PF = 0 |
| JO disp8 | Jump if OF = 1 |
| JNO disp8 | Jump if OF = 0 |
| JS disp8 | Jump if SF = 1 |
| JNS disp8 | Jump if SF = 0 |
| JZ disp8 | Jump if result is zero, i.e. ZF = 1 |
| JNZ disp8 | Jump if result is not zero, i.e. ZF = 0 |

Key concept (slide): signed/unsigned conditional branches check flags; if the conditions are true, program control is transferred to a new memory location in the same segment by modifying the content of IP.

Iteration Control Instructions (execute instructions a number of times):

| Instruction | Meaning |
| --- | --- |
| LOOP | Loop a group of instructions until CX = 0 |
| LOOPE / LOOPZ | Loop till ZF = 1 and CX ≠ 0 |
| LOOPNE / LOOPNZ | Loop till ZF = 0 and CX ≠ 0 |
| JCXZ | Jump to the provided address if CX = 0 |

MOV CX, 06H label: ADD AL, BL LOOP label ; decrements CX, loops if CX ≠ 0

# 10. ASSEMBLER DIRECTIVES (Pseudo Instructions)

Definition: Instructions to the assembler regarding the program being executed. They control the generation of machine codes and organisation of the program, but NO machine codes are generated for assembler directives. Also called pseudo instructions.

Used to:

•  specify the start and end of a program

•  attach values to variables

•  allocate storage locations to input/output data

•  define start and end of segments, procedures, macros, etc.

List of directives in the slides: DB, DW, SEGMENT, ENDS, ASSUME, ORG, END, EVEN, EQU, PROC, FAR, NEAR, ENDP, SHORT, MACRO, ENDM.

## 10.1 DB (Define Byte)

•  Defines a byte type (8-bit) variable; reserves specific memory locations for each variable.

•  Range: 00H–FFH for unsigned; 00H–7FH positive; 80H–FFH negative.

•  General form: variable DB value/values

•  Example:

LIST DB 7FH, 42H, 35H ; three consecutive memory locations reserved for LIST, ; each holding the specified initial value

## 10.2 DW (Define Word)

•  Defines a word type (16-bit) variable; reserves two consecutive memory locations for each variable.

•  Range: 0000H–FFFFH unsigned; 0000H–7FFFH positive; 8000H–FFFFH negative.

•  General form: variable DW value/values

•  Example:

ALIST DW 6512H, 0F251H, 0CDE2H ; six consecutive memory locations reserved

## 10.3 Remaining Directives

| Directive | Meaning |
| --- | --- |
| SEGMENT | Indicates the beginning of a code/data/stack segment. General form: Segnam SEGMENT … Segnam ENDS |
| ENDS | Indicates the end of a code/data/stack segment |
| ASSUME | Informs the assembler which segment name should be used for a specific segment register. ASSUME segreg : segnam, … — e.g. ASSUME CS: ACODE, DS: ADATA tells the assembler that program instructions are stored in segment ACODE and data in ADATA |
| ORG | Origin — assigns the starting address (EA) for a program/data segment. ORG 1000H → statements following it are stored from effective address 1000H |
| END | Terminates a program; statements after END are ignored |
| EVEN | Informs the assembler to store program/data segment starting from an even address |
| EQU | Equate — attaches a value to a variable. LOOP EQU 10FEH → value of variable LOOP is 10FEH |
| PROC / ENDP | Beginning / end of a procedure |
| NEAR | Intrasegment call (within the same code segment) |
| FAR | Intersegment call (different code segment) |
| SHORT | Reserves one memory location for an 8-bit signed displacement in jump instructions: JMP SHORT AHEAD |
| MACRO / ENDM | Beginning / end of a macro: macroname MACRO [Arg1, Arg2 …] … ENDM |

Worked ORG/EVEN example (from slides):

_SDATA SEGMENT ORG 1200H A DB 4CH EVEN B DW 1052H _SDATA ENDS

→ Effective address of A = 1200H; B occupies 1202H and 1203H.

PROC examples:

ADD64 PROC NEAR ; assembler codes CALL/RET as NEAR call & return … RET ADD64 ENDP CONVERT PROC FAR ; assembler codes CALL/RET as FAR call & return … RET CONVERT ENDP

## 10.4 Assembler Operators

| Operator | Meaning |
| --- | --- |
| LENGTH | Tells the assembler to determine the number of elements in a named data item (string/array), e.g. MOV CX, LENGTH STRING1 loads the count of elements in STRING1 into CX. For a byte string → number of bytes; word string → number of words. Also: byte length of a label — MOV CX, LENGTH ARRAY. |
| OFFSET | Returns the 16-bit offset address of a label; the assembler replaces "OFFSET label" by the computed offset. e.g. MOV SI, OFFSET LIST |
| LEA | Load Effective Address — loads the address of a variable. e.g. LEA AX, TEST for TEST DB 23H, 40H, 44H |

# 11. PROCEDURES, MACROS AND MODULAR PROGRAMMING

## 11.1 Procedure

A procedure is a group of instructions that usually performs one task. It is a reusable section of software stored in memory once, but can be used as often as necessary.

Two types:

•  NEAR procedure: written (defined) in the same code segment that calls it → only IP changes.

•  FAR procedure: written in a different code segment than the calling segment → both IP and CS change.

Directives: PROC identifies the start of a procedure (followed by a name and NEAR/FAR); ENDP with the procedure name marks the end. PROC and ENDP bracket a procedure.

## 11.2 CALL and RET

CALL transfers execution to a procedure. It performs two operations:

1.  Stores the address of the instruction after CALL on the stack.

2.  Changes IP (Near call) or IP and CS (Far call).

Near Call (Intrasegment): 8086 decrements SP by 2 and copies IP onto the stack; then copies the address of the first instruction of the called procedure into IP.

Far Call (Intersegment): 8086 decrements SP by 2 and pushes CS; decrements SP by 2 again and pushes IP; then loads CS with the base address of the segment containing the procedure and IP with the address of its first instruction.

## 11.3 Procedure vs Macro (COMPARISON TABLE — high-yield)

| Procedure | Macro |
| --- | --- |
| Used for large groups of instructions to be repeated | Used for small groups of instructions to be repeated |
| Object code is generated only once in memory | Object code is generated every time the macro is called |
| CALL & RET instructions are used to call and return | Macro is called just by writing its name |
| Length of the object file is less | Object file becomes lengthy |
| PROC & ENDP directives are used for defining a procedure | MACRO and ENDM directives are used for defining a macro |
| More time is required for its execution | Less time is required for its execution |
| Defined as: Procedure_name PROC … Procedure_name ENDP | Defined as: MACRO-name MACRO [ARGUMENT, … ARGUMENT N] … ENDM |
| Example: Addition PROC near … Addition ENDP | Example: Display MACRO msg … ENDM |

## 11.4 Modular Programming

Modular programming is a software design technique that emphasises separating the functionality of a program into independent, interchangeable modules, such that each contains everything necessary to execute only one aspect of the desired functionality.

•  A module interface expresses the elements that are provided and required by the module; elements in the interface are detectable by other modules.

•  The implementation contains the working code corresponding to the elements declared in the interface.

•  Closely related to structured programming and object-oriented programming — all share the goal of facilitating construction of large software by decomposition into smaller pieces (all originated around the 1960s).

•  New usage: "modular programming" = high-level decomposition of the code of an entire program into pieces; structured programming = low-level code use of structured control flow; OOP = data use of objects.

# 12. 8086 INTERRUPTS

## 12.1 Sources of Interrupts (3 types)

1.  An external signal applied to the NMI or INTR input pin → (Hardware interrupt)

2.  Execution of an INTn (n = 00H–FFH) instruction → (Software interrupt)

3.  Interrupt caused by some error condition produced in the 8086 instruction execution process — (e.g. divide by zero, overflow errors)

Interrupt classification chart (from slides):

8086 Interrupts ├── Hardware Interrupts │ ├── Maskable Interrupt (INTR) │ │ → The programmer can choose to mask specific interrupts │ │ and re-enable them later │ └── Non-maskable Interrupt (NMI) │ → The programmer CANNOT control when a non-maskable │ interrupt is served │ → The processor has to stop the main program to execute │ the NMI Service Routine └── Software Interrupts (INT n) → 256 types of software interrupts, INT 00 to INT FF

INTR (pin 18) is where maskable hardware interrupts are applied; NMI (pin 17) is non-maskable.

Uses of Non-Maskable Interrupts:

•  Used during power failure

•  Used during critical response times

•  Used during non-recoverable hardware errors

•  Used as watchdog interrupt

•  Used during memory parity errors

## 12.2 Interrupt Processing — Steps performed by the 8086

If an interrupt has been requested, the 8086 processes it by performing the following series of steps:

1.  Pushes the content of the flag register onto the stack to preserve the status of IF and TF flags, by decrementing SP by 2.

2.  Disables the INTR interrupt by clearing IF in the flag register.

3.  Resets TF in the flag register, to disable the single-step (trap) interrupt.

4.  Pushes the content of CS onto the stack by decrementing SP by 2.

5.  Pushes the content of IP onto the stack by decrementing SP by 2.

6.  Performs an indirect far jump to the start of the Interrupt Service Routine (ISR) corresponding to the received interrupt.

Flow (slide version):

Main Program → Executes the Interrupt instruction → Jumps to the Interrupt Vector Table → Takes the CS and IP in the Vector Table → Pushes the existing CS and IP on the Stack → Loads the new CS and IP → Jumps to the ISR → Executes ISR → Comes back and continues the Main Program

Hardware action diagram:

Interrupt → Push flag register → Clear IF and TF → Push CS and IP → Load CS and IP → [Interrupt Service Routine (ISR) … ends with IRET] → Pop IP and CS → Pop flags register → main program continues

Remember the order: FLAGS → CS → IP pushed; IRET pops IP → CS → FLAGS.

## 12.3 Interrupt Vector Table (IVT)

•  Located at the bottom of memory: addresses 00000H – 003FFH (1 KB).

•  Holds 256 interrupt vectors, each vector = 4 bytes (2 bytes for IP + 2 bytes for CS).

•  For a vector of type n, the ISR address is stored at:

Offset = Type number × 4

Example: INT 02H → Offset = 02 × 4 = 08 = 00008H

Layout of one vector (Type 0 example):

| Memory address | Content |
| --- | --- |
| 00000H | IP low byte |
| 00001H | IP high byte |
| 00002H | CS low byte |
| 00003H | CS high byte |

IVT layout (memory address in hex):

| Address | Vector contents |
| --- | --- |
| 00000H–00003H | Type 00H — Divide by Zero |
| 00004H–00007H | Type 01H — Trap/Single step |
| 00008H–0000BH | Type 02H — NMI |
| 0000CH–0000FH | Type 03H — Break point |
| 00010H–00013H | Type 04H — Overflow |
| 00014H–0007FH | Type 05H — Reserved |
| 00080H–00083H | Type 20H — Available |
| 00084H–00087H | Type 21H — Available |
| … | … |
| 003FCH–003FFH | Type FFH — Available |

Order inside each 4-byte vector: lower 2 bytes = IP, upper 2 bytes = CS.

## 12.4 The 256 Interrupts are Divided into 3 Groups

| Group | Types | Purpose |
| --- | --- | --- |
| 1 | Type 00 to Type 04 | Used for fixed operations → called dedicated interrupts (5 total) |
| 2 | Type 05 to Type 31 | Not used by 8086; reserved for higher processors like 80286, 80386 etc. (27 reserved) |
| 3 | Type 32 to Type 255 | Available for the user → user-defined interrupts; can be H/W interrupts activated through the INTR line or S/W interrupts (224 available) |

## 12.5 Dedicated Interrupts — Types 0 to 4 (MUST MEMORISE)

| Type | Name | Description |
| --- | --- | --- |
| Type 0 | Divide by Zero Error Interrupt | Occurs when the quotient is too large to fit in AL/AX, or on divide by zero. (Example: `MOV AL,05H / MOV DL,00H / DIV DL / HLT` → "Attempted to divide by zero" error.) |
| Type 1 | Single step or Trap Interrupt | Used for executing the program in single-step mode by setting the trap flag. |
| Type 2 | Non-Maskable Interrupt (NMI) | Used for executing the ISR of the NMI pin (positive edge signal); NMI cannot be masked by software. |
| Type 3 | One-byte INT instruction interrupt | Used for providing break points in the program. |
| Type 4 | Overflow Interrupt | Used to handle any overflow error after signed arithmetic (of = 1). |

# 13. EXAMPLE PROGRAMS (write these in the exam — they carry full marks)

## 13.1 Procedure Example Program (with data + stack + code segments)

ASSUME CS:CODE, DS:DATA, SS:STACK_SEG DATA SEGMENT NUM1 DB 50H NUM2 DB 20H ADD_RES DB ? SUB_RES DB ? DATA ENDS STACK_SEG SEGMENT DW 40 DUP(0) ; stack of 40 words, all initialized to zero TOS LABEL WORD STACK_SEG ENDS CODE SEGMENT START: MOV AX, DATA ; initialize data segment MOV DS, AX MOV AX, STACK_SEG ; initialize stack segment MOV SS, AX MOV SP, OFFSET TOS ; initialize stack pointer to TOS CALL ADDITION CALL SUBTRACTION MOV AH, 4CH ; return to DOS INT 21H ADDITION PROC NEAR MOV AL, NUM1 MOV BL, NUM2 ADD AL, BL MOV ADD_RES, AL RET ADDITION ENDP SUBTRACTION PROC MOV AL, NUM1 MOV BL, NUM2 SUB AL, BL MOV SUB_RES, AL RET SUBTRACTION ENDP CODE ENDS END START

Key points to write as comments: MOV AX, DATA → MOV DS, AX because a segment register cannot be loaded with an immediate — must go through a general-purpose register.

## 13.2 Macro Example Program

ASSUME CS:CODE, DS:DATA DATA SEGMENT NUM1 DW 1000H NUM2 DW 2000H RES DW ? DATA ENDS CODE SEGMENT ADDITION MACRO NO1, NO2, RESULT MOV AX, NO1 MOV BX, NO2 ADD AX, BX MOV RESULT, AX ENDM START: MOV AX, DATA ; initialize data segment MOV DS, AX ADDITION NUM1, NUM2, RES ; macro call — just write its name MOV AH, 4CH INT 21H CODE ENDS END START

## 13.3 Divide-by-Zero Demonstration (Emu8086)

MOV AL, 05H MOV DL, 00H DIV DL ; → divide by zero → Type 0 interrupt HLT

## 13.4 Overflow Demonstration

A signed arithmetic overflow (e.g. INT32 overflow in the slides' example) triggers the Type-4 Overflow interrupt handling path — the same principle as INTO/OF flag handling.

# 14. COMMON EXAM QUESTIONS ON THIS MODULE (with 1-line answers)

| Question | Answer skeleton |
| --- | --- |
| Draw and explain the architecture of 8086. | BIU (segment registers CS/DS/SS/ES, IP, dedicated 20-bit adder, 6-byte queue) + EU (ALU, general registers AX/BX/CX/DX, SP/BP, SI/DI, flag register) — explain pipelining (fetch & execute simultaneously). |
| Explain the pin diagram of 8086 / difference between minimum and maximum mode pins. | Use §4 tables: AD0–AD15, A16/S3–A19/S6, BHE/S7, MN/MX, RD, READY, RESET, CLK, INTR, NMI, TEST; min-mode ALE, DEN, DT/R, M/IO, WR, INTA, HOLD, HLDA; max-mode S0–S2, QS0–QS1, LOCK, RQ/GT0–RQ/GT1 + MN/MX = 1 min / 0 max. |
| How is the physical address calculated? | PA = (Segment × 16₁₀) + Offset; example 89AB:F012 = 98AC2H. |
| List and explain the addressing modes with examples. | All 12 modes with the worked EA/BA/MA examples of §8. |
| Explain the flag register. | Bits: CF(0), PF(2), AF(4), ZF(6), SF(7), TF(8), IF(9), DF(10), OF(11). |
| Difference between procedures and macros. | Use the 7-row comparison table of §11.3. |
| Difference between NEAR and FAR call. | NEAR: only IP pushed/changed; FAR: CS then IP pushed, both changed. |
| What is the IVT? How is the vector address calculated? | 00000H–003FFH, 256 vectors × 4 bytes; Offset = Type × 4; lower 2 bytes = IP, upper 2 = CS. |
| Explain the interrupt processing steps of 8086. | Push flags → clear IF → reset TF → push CS → push IP → indirect far jump to ISR (§12.2). |
| Explain types of interrupts in 8086. | Hardware (INTR/NMI) + Software (INT n) + error conditions; dedicated types 0–4, reserved 5–31, available 32–255. |
| Explain string instructions. | MOVS/CMPS/SCAS/LODS/STOS + REP/REPZ/REPNZ, SI/DI with DS/ES, DF effect. |
| Explain assembler directives. | DB, DW, SEGMENT, ENDS, ASSUME, ORG, END, EVEN, EQU, PROC, ENDP, FAR, NEAR, SHORT, MACRO, ENDM. |
| Explain MUL/DIV instructions. | Byte: AL×operand→AX; Word: AX×operand→DX:AX. DIV: AX÷r8 → AL quotient, AH remainder; DX:AX÷r16 → AX quotient, DX remainder. |

# 15. LAST-MINUTE REVISION CRAM SHEET

Numbers to remember:

| Item | Value |
| --- | --- |
| 8086 released / word size | 1978 / 16-bit |
| Transistors | 29,000 |
| Package / supply / clock duty cycle | 40-pin DIP / +5 V / 33 % (asymmetric) |
| Address bus / memory | 20-bit / 1 MB (2²⁰) |
| Data bus | 16-bit |
| Segment size | 64 KB (4 segments active = 256 KB) |
| Instruction queue | 6 bytes FIFO |
| Physical address formula | (Seg × 16₁₀) + Offset |
| IVT location & size | 00000H–003FFH, 1 KB, 256 vectors × 4 bytes |
| Vector address | Type number × 4 |
| Interrupt groups | 05 dedicated (0–4), 27 reserved (5–31), 224 available (32–255) |
| RESET must be high for | at least 4 clock cycles |

Flags: CF bit 0, PF 2, AF 4, ZF 6, SF 7, TF 8, IF 9, DF 10, OF 11.

Modes: MN/MX = 1 → Minimum (no coprocessor); MN/MX = 0 → Maximum (multiprocessor, needs 8288).

Chips: 8284 = clock generator, 8282 = latch (address demux with ALE), 8286 = transceiver (data bus), 8288 = bus controller (max mode).

If you can explain only two things, explain: (a) BIU/EU + pipelining, and (b) the 12 addressing modes with one example each.

Prepared from the Module 2 lecture slides (ECE3004 – MPMC). Diagrams referred to in the slides should be reproduced alongside these notes: BIU/EU block diagram, register/flag layout, segment-register memory map, ALE/8282/8286 demultiplexing circuit, shift/rotate bit diagrams, IVT map, and interrupt processing flowchart.